# E-money

From: Ramya Radhakrishnakumar, Sushant Sharma and Vinokkumar Uthayakumar
Discipline: Computer Engineering Technology
Date: April 22, 2018

# Declaration of Joint Authorship

We acknowledge that this report to be assessed is a group work by Ramya Radhakrishnakumar, Sushant Sharma and Vinokkumar Uthayakumar. The entire report and the project itself is prepared with the group’s consent and has almost our own ideas and words. All the other references and ideas invoked in this report are confirmed and are provided in the bibliography session. Ramya Radhakrishnakumar worked on the Hardware and the Technical aspect, Sushant Sharma handled the Database connectivity and Vinokkumar Uthayakumar on the other hand focused on developing the mobile application and the establishing connectivity between the sensors.

# Approved Proposal

Executive Summary

As a student in the Computer Engineering Technology program, I will be integrating the knowledge and skills I have learned from our program into this Internet of Things themed capstone project. This proposal requests the approval to build the hardware portion that will connect to a database as well as to a mobile device application. The internet connected hardware will include a custom PCB with the following sensors and actuators user login/sign up information, fingerprint data and Banking account information. The database will store #REF!. The mobile device functionality will include It enables us to handle our banking needs and also lets us generate  the QR-code for the money we intend to send/transfer/pay. and will be further detailed in the mobile application proposal. I will be collaborating with the following company/department Humber college parts crib, prototype laboratory, Humber library, toronto public liberary and A1 electronics.. In the winter semester I plan to form a group with the following students, who are also building similar hardware this term and working on the mobile application with me Ramya Radhakrishnakumar, Vinokkumar Uthayakumar and Sushant Sharma.. The hardware will be completed in CENG 317 Hardware Production Techniques independently and the application will be completed in CENG 319 Software Project. These will be integrated together in the subsequent term in CENG 355 Computer Systems Project as a member of a 2 or 3 student group.

Background

The problem solved by this project is Certain banks have limitations such as one can use his/her bank card for free only for certain number of transactions in a month. If someone has to use more than the limited transaction allotted in a month then he/she might be charged an interest. To avoid this problem, we have come up with an idea which generates a QR code for the money one wants to transfer.. A bit of background about this topic is We thought of E-Money transfer application because we wanted to reduce the use of card to do transaction or going to a bank to send money to a person/a merchandise whom we owe money. We wanted to enhance the security when transferring money via the internet. We have decided to use fingerprint reader in our device to fulfill security requirement. This project is better than money transfer online through website because one just needs a QR scanner and a fingerprint reader to accept the money rather an answer for a security question. Using the QR code makes the transfer quicker compare to online transfer. By generating the QR code the recipient scans the code. This displays a small information containing who sent the money, who will be receiving it and the amount being sent. Doing so will reduce multiple payments to the same recipient. Now the person who receives the money can login to his account after authenticating access using his fingerprint to check if the money was deposited in his account..

Existing products on the market include [1]. I have searched for prior art via Humber’s IEEE subscription selecting “My Subscribed Content”[2] and have found and read [3] which provides insight into similar efforts.

In the Computer Engineering Technology program, we have learned about the following topics from the respective relevant courses:

- Java Docs from CENG 212 Programming Techniques in Java,

- Construction of circuits from CENG 215 Digital and Interfacing Systems,

- Rapid application development and Gantt charts from CENG 216 Intro to Software Engineering,

- Micro computing from CENG 252 Embedded Systems,

- SQL from CENG 254 Database with Java,

- Web access of databases from CENG 256 Internet Scripting; and,

- Wireless protocols such as 802.11 from TECH152 Telecom Networks.

This knowledge and skill set will enable me to build the subsystems and integrate them together as my capstone project.

Methodology

This proposal is assigned in the first week of class and is due at the beginning of class in the second week of the fall semester. My coursework will focus on the first two of the 3 phases of this project:
	Phase 1 Hardware build.
	Phase 2 System integration.
	Phase 3 Demonstration to future employers.

Phase 1 Hardware build

The hardware build will be completed in the fall term. It will fit within the CENG Project maximum dimensions of 12 13/16" x 6" x 2 7/8" (32.5cm x 15.25cm x 7.25cm) which represents the space below the tray in the parts kit. The highest AC voltage that will be used is 16Vrms from a wall adaptor from which +/- 15V or as high as 45 VDC can be obtained. Maximum power consumption will be 20 Watts.

Phase 2 System integration

The system integration will be completed in the fall term.

Phase 3 Demonstration to future employers

This project will showcase the knowledge and skills that I have learned to potential employers.

The brief description below provides rough effort and non-labour estimates respectively for each phase. A Gantt chart will be added by week 3 to provide more project schedule details and a more complete budget will be added by week 4. It is important to start tasks as soon as possible to be able to meet deadlines.

We are planning to purchase jumper wires, an acrylic case for sensors and raspberry, android phone, breadboard, led display, data cable wire to connect the phone to the pi. Wires to connect all the sensors together. And also need to purchase the firebase(database) membership.

Concluding remarks

This proposal presents a plan for providing an IoT solution for We will be working together consistently as a team every week. We will be getting help from Mr.Kelly and Mr.Vlad to get better understanding to things. Safety is our first priority and we will make sure that we do not take any dangerous step. Also we would like to work on the software part so that it collaborates with our hardware needs.. This is an opportunity to integrate the knowledge and skills developed in our program to create a collaborative IoT capstone project demonstrating my ability to learn how to support projects such as the initiative described by [3]. I request approval of this project.

# Abstract

The peculiarity of the mobile application is that one can transfer, pay and also manage their bank account all in one. It is something like a banking application but has more features to it such as one does not have the need to go to a bank to transfer money to someone far away also the user can pay merchandised bills using this application.  E-money application needs sufficient amount of memory and access to the cellular data/Wi-Fi. The ideology of this project is to save time, carry transactions and merchandize bill payments with ease. Also, this application can prevent investing money for purchasing credit cards. This application is available for android mobile OS only for time being.

Table of Contents

Declaration of Joint Authorship	iii

Approved Proposal	iv

Executive Summary	iv

Background	v

Methodology	v

Concluding remarks	vii

Abstract	ix

1.	Introduction	13

2.	Project Description	14

2.1	Problem	14

2.2	Rationale Behind Project	14

2.3	Project scope	14

2.4  Software Requirement Specifications	15

2.4.1 Purpose	15

2.4.2 Product Perspective	16

2.4.3	Product Functions	17

2.4.4 User Classes and Characteristics	17

2.4.5 Operating Environment	18

2.4.6 Design and Implementation Constraints	18

3 External Interface Requirements	19

3.1 Hardware Interfaces	19

3.2 Software Interfaces	19

4. Mobile Application Features	19

4.1 Case Scenarios	19

4.2 Database	20

Database 1:	21

Database 2: (yet to be modified)	22

4.3 Mobile Application	22

5. Project Overview	23

5.1 Fingerprint Sensor	23

5.1.1 Introduction	23

5.1.2 Invoice/Bill:	24

5.1.3 Budget	25

5.1.4 Time Commitment	25

5.1.5 Mechanical Assembly of my Fingerprint sensor	26

5.1.6 Power up	27

5.1.7 Installation of the Raspberry Pi Fingerprint Library	27

5.1.8 Unit Testing	28

5.1.9 Production testing	28

5.2 QR Scanner	29

5.2.1 Introduction	29

5.2.2 Bill of Materials Budget	29

5.2.3 Time Commitment	30

5.2.4 Mechanical Assembly	30

5.2.5 Unit Testing	31

5.2.6 Production Testing	31

6. Problems Encountered	31

6.1 Data Communication	31

6.2 LCD Display	31

6.3 Mobile Application	32

6.4 QR code email	32

7. Progress Reports	32

7.1  Report 1	32

7.2 Report 2	35

7.3 Report 3	37

8. Conclusions	39

9. Recommendations	39

10. Technical References/Bibliography	39

11. Appendices	40

# Introduction

A significant problem being faced by customers these days is that, every customer has to go to the bank, wait in line to be helped and then consult our needs with the representative to perform his/her banking needs such as monitoring the bank account, transfer money, pay bills, security and credit card fraud. The e-money application can overcome these problems.

To make this application work, it requires certain information such as Acc.no, name and password to be stored in the database which will then be accessed via both a hardware and Smartphone application, providing on-the-go banking experience. This application is convenient and efficient keeping user friendliness as a key to work with the application.

Since our mobile application needs a Wi-Fi or cellular data to send an email to the recipient, the whole set-up should be placed in an environment that supports either Wi-Fi or cellular data.

# Project Description

### Problem

The problem encountered is, the user has to go to a bank to meet their daily banking needs such as transfer money, managing bank account and also pay merchandiser bills. The most current existing mobile applications do not have all the above features and it seems as though the user need to download more than one mobile application. This intern leads to large memory consumption and security issues. There are few banking mobile applications but these have few limitations. One of the major problem is that if the user enters the wrong account number followed by the wrong routing number it will cause the money to go into another account. Also, the user needs to double check the amount the user intends to send.

## Rationale Behind Project

In order to solve the above-mentioned problems, the team came up with the idea of E-money and worked towards this project. It is simply an inexpensive, user friendly, very specific, sophisticated and realistic mobile application being named the E-money. From the very name the mobile application carries all its functions using the internet. This mobile application will be the key to security for your banking information and also it is free of charge. It helps in transferring, paying and also managing accounts from one place.

## Project scope

E-Money is so beneficial since the users need to have to waste time going into a physical bank and wait for a teller to help them out to transfer money. This can also be out stretched and used to pay the application user’s mortgage or car loan. This is much quicker as a transfer processed by the bank takes a minimum of 2 business days to transfer any amount to any account. It is something like a direct deposit, here the user need not be physically present in the bank to perform the deposit. The security is taken over by the fingerprint reader. This simply accesses the bank database and matches the finger print to the email ID along with the card number and then pulls out the banking information. By doing so, there won’t be any miscellaneous handle of banking data’s and only the authorised user can only handle the information.

Since the intention is to produce an user-friendly and inexpensive mobile application, some of the high level security features and also banking information has been voided. Like, the user can register by using the email ID and the intended finger print. But the user need not provide the card number of the card that is issued by the bank. Also the user need not prove other personal details such as photo ID, home address, contact number, etc. . Likewise, any user can register into this application regardless of having an account in that particular bank, since that requires a well maintained banking database which keeps updating as soon as bank card is issued or expired. It also provides access to the QR code without the need to enter a passcode to open the file sent via email to transfer money to the recipient.

## 2.4  Software Requirement Specifications

## 2.4.1 Purpose

The hardware products which are going to be included in our project are fingerprint sensor and QR code scanner. The software products are firebase and android studios. The whole system will be built upon the basis of these products. This system can be further divided in two subsystems such as hardware and software aspects. The purpose for this project is to decrease the use of cash or credit/debit cards and use the money generated via QR code. The reason for using QR code is that we don’t have to worry about losing cash or even if we forget our wallet at home we can use our cell phone to pay with QR code.

## 2.4.2 Product Perspective

This product is basically a replacement of the existing banking applications and apple pay/android pay. This is built keeping security as the primary key. The application is designed in a way that the user can perform or manage transactions as well as pay their bills all in one. The following schematic gives a clear understanding:

Case 1 & 2:

E-money application

- Transfer money

- Pay using QR scanner

DB

Generate the code and let the merchandiser scan it

Generate QR code and send the code through an email to the recipient

Payment successful

QR scanner scans the code and deposits the money

Case 3:

Enter Acc.no: 1234567890

Account Balance: $3250.75

Credit: $200

Scan

Finger Print sensor

## Product Functions

* The Code scanner is used to scan the generated code.

* Mobile application is used to generate the QR code.

* The Fingerprint sensor is used for security purpose.

## 2.4.4 User Classes and Characteristics

Almost 90% of world’s population has their day starting with an android smart phone. This project is intended for people having access to their bank account and smartphone. In this era everything is possible via a smartphone for instance, one can order food, taxi, etc. using various android apps. According to e-money the use may vary from an employer to a merchandiser. This application is user friendly for both and is designed in such a way that both has access to all facilities provided by the app itself. People use online banking since one can manage monthly statements and perform transactions with ease. Whereas the merchandiser has to create a default account so that the payment made to them by the customer gets deposited in it.

## 2.4.5 Operating Environment

The environment such as weather, location etc., is not an asset for the e-money. The whole hardware is not legitimately big and hence it is easy to carry around in the parts kit itself. The application is intended for android operating system and hence it is not available for other mobile operating systems. The application collides with the fingerprint sensor to give permissions to access the user’s banking account. The barcode scanner plays a part along with the software application to perform transactions. This is how it is being organized to make the hardware and software work together.

## 2.4.6 Design and Implementation Constraints

The database incorporated in our product contains bank account info such as the ten-digit account number, account types like chequing account and savings account. The database will also include information about user login which will have the username as the account/card number, the password field as well as fingerprint for enhanced security. If the user does not have an account then they can register and make an account in order to login. For registering the user will be asked to enter their first name, last name, email id, cell phone number, account number/username, fingerprint and password. All this information will the stored in database and will be needed in order to login.

# 3 External Interface Requirements

## 3.1 Hardware Interfaces

The hardware interfaces needed for connecting our components differ from sensor to sensor. QR code scanner is going to be connected using a USB interface with the raspberry pi. The fingerprint scanner is connected through the TTL to USB convertor with the pi.

## 3.2 Software Interfaces

The database incorporated is firebase and the operating system used is Android studios (lollipop 22). The mobile application was conveniently developed with the help of Android studios which uses tools and libraries from the internet. The services needed are an internet connection, access to email protocol and converting QR code to image file and finger print sensor. Also some python code was incorporated to showcase a presentable GUI which will ease user interface.

# 4. Mobile Application Features

## 4.1 Case Scenarios

Case 1: e-mail Transfer:

In the user interface there is an option called send. Once send is selected it prompt the user to enter the amount to which we want to transfer. This then generated the QR code with the Acc.no and amount. This is packaged into a unique number. This is used only for transferring.

Now the recipient who receives the email with the unique number will be able to deposit the money if he takes the barcode to the bank. The bank who has the barcode scanner scans the code and deposits the money to the recipient’s account.

Case 2: Payment:

In this case the user will have access to generate the QR code for the amount he owes the merchandiser. This is packaged into the unique number (varies from the 1st case). This code is scanned by the merchandiser. Now the amount gets deposited directly to the merchandiser’s default account.

Case 3: Account Detail:

Now if the user wants to check the Account information he has to enter his Acc.no and scan the fingerprint associated with the bank account. The fingerprint is basically used for security purposes.

### 4.2 Database

The parent database is controlled by the bank which has details about the users such as name, card number, account balance, transaction history along with the fingerprint enrolled with the application. Also, we have incorporated two separate child database for the need of the mobile application. One database is for the mobile application user where the login credentials such as Acc. Balance, card number, email, password and fingerprint of that particular user is used to enroll or register with the application. Only if the information matches it allows to access the account. If it does not match it won’t allow permission to access the account. The second database contains the merchant (Tim-Hortons, Subway. etc) database. This is like a merchant’s account where information such as Acc. Balance of the merchant is available. Along with the account balance there is some additional information such as payment details of the customer and also the time stamp of when and which location the payment was performed. The time stamp is implicated in both the child process because it can be easy to check any miscellaneous payment or security issues

### Database 1: (Bank account)

### Database 2: (Merchant)

### 4.3 Mobile Application

In the application we added a navigation bar that uses fragments to switch between each layout between screens. Some of the methods/functions added are overview, pay, transfer, login and enroll. Every option has a short description of what it leads to by selecting it. We also added layouts for each of those methods that are mentioned above. We added customized draw able images and icons so that it looks more interactive. Some libraries are used for the firebase connectivity with the application which retrieves information from the database.

# 5. Project Overview

## 5.1 Fingerprint Sensor

## 5.1.1 Introduction

E-money transfer is a convenient way to send, receive or pay bills in store using a simple QR code generator. We are using a finger print sensor in collaboration with the software app. The sensor is used as a safety measure to prevent loss of information. In order to access your account, you need to login using your login credentials including the finger print pattern that is registered along with your bank account in the E-money application. Basically, the finger print lets you enroll, delete, search or generate a picture of your finger. To do this we used a raspberry pi along with a usb to ttl convertor and jumper wires.

## 5.1.2 Invoice/Bill:

Ordered the sensor from Adafruit website. Also ordered raspberry pi, USB to TTL and jumper wires from Amazon. It roughly took a week to reach.

### 5.1.3 Budget

- The Fingerprint sensor (751) costed $64.00.

- The Raspberry pi costed $99.99.

- The USB to TTL convertor costed $6.99.

- The Jumper wires costed $5.99

## 5.1.4 Time Commitment

The time commitment was a cake walk since I created a schedule during the start of my semester.

It really was one reason that we organized the work properly and was able to achieve the milestones as intended.

Setting up the hardware must take around 20 minutes provided if you have the equipment’s and have previous knowledge of the hardware. We took roughly 1 week to set it up. We used python code to make the sensor work. The testing should take around 10 minutes, but actually took time to read the code and understand what was actually in the code. It took 3 days to work on understanding the code and then successfully executed.

## 5.1.5 Mechanical Assembly of my Fingerprint sensor

1. Connect the fingerprint to the USB to TTL convertor as follows:

2. This is the pinout of the USB to TTL.

3. The pinout of the fingerprint sensor.

4. Connections using the jumper wires:

*Connect the GN of the sensor to the GN of the TTL converter.

*Connect the RX of the sensor to the TX of the TTL converter.

*Connect the TX of the sensor to the RX of the TTL converter.

*Connect the VCC of the sensor to the VCC of the TTL converter.

5. Then connect the sensor through the USB port of the Raspberry Pi.

## 5.1.6 Power up

So once assembled the hardware part, then connected my mouse and keyboard along with the charger cable to my pi and then turned it ON. Also we need not connect the Ethernet cable since we already configured the pi to connect it to the Wi-Fi. We were able to see the sensor light up. You will be able to achieve this if your hardware connection is perfect.

## 5.1.7 Installation of the Raspberry Pi Fingerprint Library

We need to go to the root for some commands of the installation. Then we started a terminal session and typed the following:

sudo bash

Now add the necessary package sources:

wget -O - http://apt.pm-codeworks.de/pm-codeworks.de.gpg | apt-key add -

wget http://apt.pm-codeworks.de/pm-codeworks.list -P /etc/apt/sources.list.d/

Then had to update the available packages and install the Python library as follows:

apt-get update

apt-get install python-fingerprint –yes

Now the packages are installed and ready for testing.

## 5.1.8 Unit Testing

1. As mentioned above the package has files for storing a new fingerprint, reading out and deleting stored fingerprints. Now we tested the enrolling option by issuing the following:

python2 /usr/share/doc/python-fingerprint/examples/example_enroll.py

2. The terminal asks you to place the finger you want to enroll on the sensor, wait for the message "remove finger" and then again "place the same finger".

Now the finger is given an ID and it is enrolled.

3. Now we test if the finger is recognized. So we issued the following script:

python2 /usr/share/doc/python-fingerprint/examples/example_search.py

4. Then put the finger on it again. If the fingerprint on the Raspberry Pi is detected, a message like this appears:

Currently stored templates: 2

Waiting for finger...

Found template at position #1

The accuracy score is: 90

SHA-2 hash of template: 3aa1b01149abf0a7ad0d7803eaba65c22ba084009700c3c7f5f4ecc38f020851

## 5.1.9 Production testing

The testing was successful. We were was to make the sensor work by enrolling the finger, delete the finger by giving the appropriate ID, search for a fingerprint that was enrolled and retrieve the fingerprint image. This is a very reliable for security purposes.

## 5.2 QR Scanner

## 5.2.1 Introduction

This project is about e-money transfer by using Qr code sensor. The components that have been used for this project are raspberry pi 3 and Qr code sensor. The raspberry pi comes with a case, power adapter and micro sd card with raspbian OS. A monitor, mouse and a keyboard is required to operate raspberry pi, unless you have laptop to connect it with. To connect a laptop with the pi, you have to configure specific settings accordingly which is not covered in the following instructions. The qr code sensor is connected through a usb with raspberry pi. Recommended libraries have been used to power up the sensor. I plan to demonstrate transactions of an amount of money using the qr code sensor. It can be used in stores to buy stuff and to transfer money. Building this project should require a couple of hours if you follow these instructions.

## 5.2.2 Bill of Materials Budget

We ordered raspberry pi from amazon and it arrived within three days as we used amazon prime shipping. For the most part, we used QR code scanner and raspberry pi. QR code sensor can be used to scan both barcodes and QR codes which is one of the reasons they are more expensive compared to barcode scanners. They can also scan both 1 dimensional and 2 dimensional codes and are equipped with laser.

## 5.2.3 Time Commitment

Once we received the equipment, it took approximately 2 weeks to complete the project. This period includes the time that we spent on researching and building the sensors in order for them to operate in the way we wanted them to. The QR code scanner is connected to the pi through a USB interface which took about 2 minutes to connect and test. However, the part that took the most amount of the time was creating the python code.

## 5.2.4 Mechanical Assembly

We did not need any extra electrical equipment such as wires, motors, resistors, capacitors etc. The assembly of this project requires raspberry pi, power adaptor and QR code scanner with a USB interface. Just used a mouse, keyboard and monitor to connect the pi and to test my sensor.

## 5.2.5 Unit Testing

When we first received the sensor, we were testing it by connecting it to my powered up pi. Initially, was just scanning randomly generated QR codes and barcodes from the internet. And every time we scanned, the screen we were on, happened to scroll down automatically. So, thought that the scanned item was being stored somewhere on the SD card of the pi. However, the attempt was unsuccessful. Then thought that maybe it would be stored in a text editor like notepad or MS word. So we scanned while opening a text editor and the code was stored in the file.

## 5.2.6 Production Testing

Then we developed the Python code for the intended function to be carried by the sensor. The developed code works in a way that it is connected to the firebase database account. Once the QR code generated by the application is scanned it will recognize the amount that was stored in the QR code and it will add that amount to the linked bank account in the firebase database.

# 6. Problems and Approaches

### 6.1 Data Communication

The mobile application has many fragments. One of the fragment is used to get the user information. This asks the user to enter the card number, email ID and fingerprint. The issue was the application being connected to the Firebase database was not able to fetch the account information despite giving the correct credentials.

Also issue with the python code is something about the version. It is an in built function of the firebase which may not compile the python code if the version of the python code and firebase is outdated. The python code will not be issued or compiled resulting in not displaying the GUI. So it is always wiser to have the same working version for both the firebase and the python code to avoid such minor errors. This intern leads to not fetching the data stored in the firebase database which is a serious issue. Once the version was changed, the python code was able to access the firebase database and present the GUI.

### 6.2 LCD Display

A Liquid Crystal Display was incorporated in the project. The LCD’s purpose was to present the GUI of the project. But the issue was that, it wasn’t turning on. But the touch of the screen happened to work. The team then came up with a conclusion that it was a hardware issue. After certain encounters and learning phases the problem was with the adapter of the LCD. The LCD required a minimum voltage of 5V but the adapter that was used only supplied 3.3V. Once the adapter was replaced, the LCD was up and running displaying the GUI once the python code was established.

### 6.3 Mobile Application

The mobile application had a difficulty which intern comes under software issues. The buttons and variables are used in the fragments. These buttons and variables couldn’t be passed to other fragments. This was the issue, one solution was to bundle the buttons or variables. The procedure is to bundle these buttons or variables into one and then pass this bundle as indented to other fragment. Another solution used is also similar to the previous one. Here the only difference is we bundle the object. And this bundled object will be passed to another fragment. The issue with the second solution is that it sometimes returns a null. The 1st solution is more reliable than the second.

### 6.4 QR code email

One of the case scenarios is that using the mobile application the user can send money to another user whom he owes. So to perform this transfer it requires the sender to generate the QR code and send that code over email. Initially here the issue was QR code was not transmitted via email to the receiver. It keeps asking the sender to save the QR code to the images folder of the phone. Only after saving the QR code in the image folder the user was able to email it. Then after numerous trouble shooting it was fixed by incorporating some libraries into the Android code. Now the QR code need not be saved into the image folder. It can be created and sent directly by creating the QR code for the indented transfer and entering the receiver’s email ID.

# 7. Progress Reports

## 7.1  Report 1

Ramya Radhakrishnakumar < ramyarkkumar27@gmail.com >

To: Austin Tian < Austin.Tian@humber.ca >

Cc: Sushant Sharma < sushantsharma88888@gmail.com >, Vinokkumar Uthayakumar<ukvino3@gmail.com >

Dear Austin Tian,

We have decided student A to be Ramya Radhakrishnakumar, student B to be Sushant Sharma and student C to be Vinokkumar Uthayakumar. This email is to update the progress shown in e-money since week 3.

Since our very idea of the project has been overlooked, few changes had to be made to the mobile application. So we dedicated more attention towards it. Vino worked on the changes to be made in that application and some of the modifications are; we added a navigation bar that uses fragments to switch between each layout between screens. This actually gives a proper flow for the application and also made it a bit more user friendly now than before. Some of the methods/functions added are contact, enrolment, overview, pay and transfer. We also added layouts for each of those methods that are mentioned above. Moving on to the database part controlled by Sushant, the database has been created successfully and we tried to connect it to the mobile application. It wasn’t connecting initially because of some technical issue. We spent more time on fixing the issue and now we are able to connect the database to the mobile application. We are able to interact between the application and the database now. Ramya made sure if the hardware/sensors incorporated in this project are in proper working conditions. We are yet to work on the hardware and database connection this week. Also we carried out some research on the hardware parts that might be needed in the future.

So far there is no change in the finical status of the project. It remains the same. Further updates on the budget are to be seen down the following weeks.

As per the project schedule we have completed almost as intended until week 5, yet the major milestone is yet to be completed and that is the connection of the database to the hardware (QR scanner and Fingerprint sensor) which we are planning to work on week 6 and also we might get help from Kelly in the prototype laboratory.

Links to the Media added on GitHub (https://github.com/RamyaRadhakrishnakumar/ceng355):

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/PastedGraphic-1.png

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/PastedGraphic-2.png

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/Screen%20Shot%202018-03-05%20at%202.04.40%20PM.png

Some of the websites used as reference for the application and database development are as follows:

https://www.youtube.com/watch?v=F6UWb9FNnj4

https://stackoverflow.com/questions/22882074/download-image-and-its-associated-info-through-qr-code-on-android-app

https://stackoverflow.com/questions/45967649/firebase-authentication-error-in-android

https://firebase.google.com/docs/auth/android/google-signin

https://developer.android.com/studio/write/firebase.html

https://firebase.google.com/docs/database/android/start/

Yours sincerely,

Ramya Radhakrishnakumar, Sushant Sharma & Vinokkumar Uthayakumar

Humber College

School of Applied Tech

## 7.2 Report 2

Sushant Sharma < sushantsharma88888@gmail.com >

To: Austin Tian < Austin.Tian@humber.ca >

Cc: Ramya Radhakrishnakumar < ramyarkkumar27@gmail.com >, Vinokkumar Uthayakumar<ukvino3@gmail.com >

Dear Austin Tian,

This email is an overview of our progress during week 6 updated by student B.

This week we worked on the application and database connectivity with the hardware as well as the app. Sushant and Vino worked on validating the login, registering and authentication with the firebase database. Ramya spent some time researching on how to connect the firebase database with our raspberry pi, so that we can store our sensor’s scanned data into it.

We created a separate database for Merchant. We are able to store the value scanned by QR code scanner and add it to the merchandiser database.

Some of the problems/challenges encountered this week was, we tried to hide the navigation bar after the user logs in so that they stay logged in even if they click on different fragment/activity. We are also looking for a way to hide the login fragment when the user is already logged in until they click logout button.  We also imported the libraries for python to connect it to the database. We are up to date according to the schedule.

The financial status is the same as before. Most likely it will not change as we have acquired all the necessary equipment.

Links to the Media added on GitHub (https://github.com/RamyaRadhakrishnakumar/ceng355):

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/week6/transfer.PNG

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/week6/enrol.PNG

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/week6/login.PNG

Some of the websites used as reference for the application and database development are as follows:

https://pypi.python.org/pypi/python-firebase/1.2

https://www.raspberrypi.org/forums/viewtopic.php?t=183455

https://www.youtube.com/watch?v=BJfVoaifnzc

https://repl.it/repls/WorrisomeArcticBlock

http://python-textbok.readthedocs.io/en/1.0/Variables_and_Scope.html

Yours sincerely,

Ramya Radhakrishnakumar, Sushant Sharma & Vinokkumar Uthayakumar

Humber College

School of Applied Tech

## 7.3 Report 3

Vinokkumar Uthayakumar<ukvino3@gmail.com >

To: Austin Tian < Austin.Tian@humber.ca >

Cc: Ramya Radhakrishnakumar < ramyarkkumar27@gmail.com >, Sushant Sharma <sushantsharma88888@gmail.com >

Dear Austin Tian,

This email is an overview of the progress during week 8 and week 9 updated by student C.

In the last two weeks, work was focused on the Graphical User Interface(GUI) for the sensors. Now the GUI for QR code scanner and fingerprint programs has been created. There is a plan to use LCD display for the sensors. Also, the team is working on subtracting the money from the bank account of the user after they have made a transaction(Pay/Transfer). Also, work was done on python code for fingerprint sensor to store information in the database based on the user’s fingerprint and email address provided.

There have been updates made to the layout of the application.

The problems being faced in the last 2 weeks are as follows, firstly the user information from the fire-base database was not being displayed. Here the user information could be seen from the database when debug is used. Finally, the information is being displayed on the overview such as the account number and account balance of the current user. Secondly, the object class is not working with fragments. The problem is, the buttons could not be transferred/passes from one class to another. The team tried fixing the issue by incorporating instance to make it work but it wasn’t successful. We are not able to call a method from one fragment to another fragment.

We are on track with the project schedule.

There is a recent purchase that adds up to the financial update of the project. An LCD display has been purchased for CAD $55.

Links to the Media added to GitHub (https://github.com/RamyaRadhakrishnakumar/ceng355):

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/qr%20code%20scanner%20gui.PNG

https://github.com/RamyaRadhakrishnakumar/ceng355/blob/master/fingerprint%20gui.PNG

https://github.com/RamyaRadhakrishnakumar/ceng355/tree/master/Layouts%20of%20the%20application

# 8. Conclusions

# 9. Recommendations

# 10. Technical References/Bibliography

https://www.youtube.com/watch?v=F6UWb9FNnj4

https://stackoverflow.com/questions/22882074/download-image-and-its-associated-info-through-qr-code-on-android-app

https://stackoverflow.com/questions/45967649/firebase-authentication-error-in-android

https://firebase.google.com/docs/auth/android/google-signin

https://developer.android.com/studio/write/firebase.html

https://firebase.google.com/docs/database/android/start/

https://pypi.python.org/pypi/python-firebase/1.2

https://www.raspberrypi.org/forums/viewtopic.php?t=183455

https://www.youtube.com/watch?v=BJfVoaifnzc

https://repl.it/repls/WorrisomeArcticBlock

http://python-textbok.readthedocs.io/en/1.0/Variables_and_Scope.html

# 11. Appendices

11.1 Python Code Bank Account

import sys

import re

import random

import math

import time

from firebase import firebase

from Tkinter import *

from random import *

root = Tk()

root.title("QR GUI Store")

root.geometry("590x410")

firebase = firebase.FirebaseApplication('https://qrmoney-1aec0.firebaseio.com')

#result = firebase.put('Transaction','Account Number',{'From': '','Account Balance': ''})

code = []

amount = []

final = []

finalbalance = []

uid = []

fbalance = []

l = Label(root, text="Please scan the QR Code: ")

l.grid(row = 0, column = 0)

entry = Entry()

entry.grid(row = 0, column = 1)

try:

while True:

def get():

'''Go and get the text from the entry'''

txt.delete(0.0, 'end')

code = (entry.get())

#amount = code.split(",")

#print amount[0]

#sentense = ("You paid", amount[1])

amount = re.split(r'[,.]',code)

sentense = ("You paid", amount[2])

print ("From: ", amount[0])

print ("To: ", amount[1])

print ("Amount: ", "$" + amount[2])

txt.insert(0.0, sentense)

uid = firebase.get('https://qrmoney-1aec0.firebaseio.com', None)

for x in uid:

if (amount[0] == x):

print("ok")

AccountBalance = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+amount[1],"/Account Balance")

fromamount = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+amount[0],"/Account Balance")

print("From (before tranfer): ", fromamount)

print("To (before transfer): ", AccountBalance)

finalbalance = float(amount[2]) + float(AccountBalance)

if(finalbalance != None):

x = randint(1,100)

ts = time.time();

#for x in range (0, 1000):

firebase.put("https://qrmoney-1aec0.firebaseio.com/"+amount[1],"/Account Balance", float(finalbalance))

firebase.put("https://qrmoney-1aec0.firebaseio.com/"+amount[1],'/Transaction' +str(x),{'From':  amount[0] ,'To':  amount[1] ,'amount': amount[2], 'Time': ts})

#for y in range (0, 1000):

fbalance = float(fromamount) - float(amount[2])

firebase.put("https://qrmoney-1aec0.firebaseio.com/"+amount[0],'Transaction'+str(x),{'From': amount[0],'To':  amount[1],'amount':amount[2], 'Time': ts})

print("Time: ", ts)

#result = firebase.put('Customer','AccountInfo',{'Account Number': '','Account Balance': ''})

firebase.put("https://qrmoney-1aec0.firebaseio.com/"+amount[0],"/Account Balance", float(fbalance)

print("Final Balance after transfer in 'TO' account", finalbalance)

print("Final Balance after transfer in 'FROM' account", fbalance)

Button(text="Deposit", command=get).grid(row = 2, column = 0)

txt = Text(root, width=45, height=8)

txt.grid(row = 4, column = 0, columnspan = 2)

root.mainloop()

except KeyboardInterrupt:

print ("\nExit")

11.2 Python Code Merchant

import sys

from firebase import firebase

from Tkinter import *

import time

import math

import random

from random import *

root = Tk()

root.title("QR GUI Store")

root.geometry("410x280")

root.configure(background='orange')

firebase = firebase.FirebaseApplication('https://qr-merchant.firebaseio.com')

AccountBalance = firebase.get('https://qr-merchant.firebaseio.com','OUR DATABASE/17150R81400/Account Balance')

print (AccountBalance)

code = []

amount = []

final = []

finalbalance = []

ts = time.time()

l = Label(root, text="Please scan the QR Code: ",background="orange")

l.grid(row = 0, column = 0)

txt1 = Text(root, width=27, height=1)

txt1.grid(row = 0, column = 1)

txt = Text(root, width=45, height=8)

txt.grid(row = 4, column = 0, columnspan = 2)

try:

while True:

code = raw_input("Please scan the QR Code: ")

amount = code.split(",")

txt1.insert(0.0, amount)

sentense = ("You paid", "$"+amount[1])

txt.insert(0.0, sentense)

finalbalance = float(amount[1]) + float(AccountBalance)

print(finalbalance)

x  = randint(1,1000)

#result = firebase.put('Customer','AccountInfo'+amount[0],{'Account Number': amount[0],'Account Balance': amount[1], 'Time': ts})

#result = firebase.put('Customer','AccountInfo'+amount[0])

result = firebase.put('Customer',str(x) + 'AccountInfo'+amount[0],{'Account Number': amount[0],'Account Balance': amount[1], 'Time': ts})

firebase.put('https://qr-merchant.firebaseio.com','OUR DATABASE/17150R81400/Account Balance', format(finalbalance, '.2f'))

firebase.put('https://qr-merchant.firebaseio.com','OUR DATABASE/17150R81400/Time', ts)

#firebase.put('https://qr-merchant.firebaseio.com','Customer/AccountInfo'+str(amount[0])'/Account Balance', float(amount[1]))

#firebase.put('https://qr-merchant.firebaseio.com','Customer/AccountInfo'+str(amount[0])'/Account Number', amount[0])

root.mainloop()

except KeyboardInterrupt:

print ("\nExit")

11.3 Python Code Fingerprint (Enroll, Search and Delete)

import time

import os

from os import path

import tempfile

import json

import hashlib

import urllib, StringIO

from random import *

from pyfingerprint.pyfingerprint import PyFingerprint

from firebase import firebase

from Tkinter import *

import numpy as np

from google.cloud import storage

import subprocess as sub

import cv2

root = Tk()

root.title("FingerPrint GUI")

root.geometry("710x400")

code = []

email = []

uid = []

n = [100]

ts = time.time();

txt = Text(root, width=30, height=15)

txt2 = Text(root, width=30, height=15)

txt3 = Text(root, width=30, height=15)

firebase = firebase.FirebaseApplication('https://qrmoney-1aec0.firebaseio.com/')

os.environ["GOOGLE_APPLICATION_CREDENTIALS"] = "/home/pi/Desktop/qrmoney-ebd1ca9fcc42.json"

client = storage.Client()

bucket = client.get_bucket('qrmoney-1aec0.appspot.com')

try:

f = PyFingerprint('/dev/ttyUSB0', 57600, 0xFFFFFFFF, 0x00000000)

if ( f.verifyPassword() == False ):

raise ValueError('The given fingerprint sensor password is wrong!')

except Exception as e:

print('The fingerprint sensor could not be initialized!')

#cent1 = ('The fingerprint sensor could not be initialized!')

#txt2.insert(0.0, cent1)

print('Exception message: ' + str(e))

exit(1)

def search():

print('Waiting for finger...')

#############################################################

#Wait that finger is read

while ( f.readImage() == False ):

pass

f.convertImage(0x01)

result = f.searchTemplate()

position = result[0]

print(position)

if ( position == -1 ):

print('No match found!')

exit(0)

else:

print('Found template at position #' + str(position))

## OPTIONAL stuff

##

## Loads the found template to charbuffer 1

f.loadTemplate(position, 0x01)

#Searchs template

############################################################################################################

print("ok1")

#giraffeBlob = bucket.get_blob("image1")

print("ok2")

#URL = giraffeBlob.public_url

print("ok3")

#req = urllib.urlopen(URL)

print("ok4")

#arr = np.asarray(bytearray(req.read()),dtype=np.uint8)

print("ok5")

#img = cv2.imdecode(arr,-1)

print("ok6")

#giraffeBlob.download_as_string()

print("ok7")

#file = cStringIO.StringIO(urllib.urlopen(URL).read())

print("ok8")

#img = Image.open(file) #img is my image from storage and  f is from user so convert now

print("ok5")

#print(img)

#cv2.imshow("image", img)

print("ok6")

############################################################################################################

imageDestination =  tempfile.gettempdir() + '/fingerprint.bmp'

########################################################

uid = firebase.get("https://qrmoney-1aec0.firebaseio.com/", None)

print("ok")

code = (entry.get())

print("ok")

email = code.split('.')

print("ok")

for x in uid:

print("ok")

#print(x)

###############################################################

if (email[0] == x):

print("ok")

test = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+email[0],"/fingerprintImage")

print(test)

if(position == test):

test2 = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+email[0],"/CardNumber")

test3 = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+email[0],"/Account Balance")

#if(test == imageDestination):

print(test)

print(test2)

print(test3)

#imageDestination =  tempfile.gettempdir() + '/fingerprint.bmp'

print('Downloading image (this take a while)...')

#f.downloadImage(test)

print("storeed")

print('The image was saved to "' + test + '".')

print(f)

print(imageDestination)

else:

#cent5 = (test)

#txt2(0.0, cent5)

print("Finger print is not matching with our datbase.....")

#root.mainloop()

else:

print("Your email id is not matching with our databse Please try agian later time")

def enroll():

sent4 = ('Waiting for finger...')

#label4 = Label(root, text="Waiting for same finger again...").grid()

print('Waiting for finger...')

while (f.readImage() == False):

pass

print ("Remove finger...")

sent2 = ("Remove finger...")

txt.insert(END, sent2)

print('Waiting for same finger again...')

#Wait that finger is read again

while ( f.readImage() == False ):

pass

#########################################################333

f.convertImage(0x02)

##########################################################

f.createTemplate()

positionNumber = f.storeTemplate()

imageDestination =  tempfile.gettempdir() + '/fingerprint.bmp'

f.downloadImage(imageDestination)

#print ('The image was saved to "' + imageDestination + '".')

############################################################################################

#for k in range 10:

x = randint(1,100)

imageBlob = bucket.blob("image" +str(x))

imageBlob.upload_from_filename(imageDestination)

#num +=1

print("ok")

############################################################################################

#loop through all the uid

code = (entry.get())

uid = firebase.get("https://qrmoney-1aec0.firebaseio.com/", None)

email = code.split('.')

for x in uid:

print(x)

# if my email and uid is matches

if (email[0] == x):

firebase.put("https://qrmoney-1aec0.firebaseio.com/"+email[0],'/fingerprintImage', positionNumber)

print('Finger enrolled successfully!')

#pass

else:

print("Your email id is not matching with our databse Please try agian later time")

def delete():

print('Currently used templates: ' + str(f.getTemplateCount()) +'/'+ str(f.getStorageCapacity()))

## Tries to delete the template of the finger

print('Waiting for finger...')

while (f.readImage() == False):

pass

print ("Remove finger...")

sent2 = ("Remove finger...")

txt.insert(END, sent2)

print('Waiting for same finger again...')

while ( f.readImage() == False ):

pass

code = (entry.get())

uid = firebase.get("https://qrmoney-1aec0.firebaseio.com/", None)

email = code.split('.')

for x in uid:

print(x)

# if my email and uid is matche

if (email[0] == x):

positionNumber = firebase.get("https://qrmoney-1aec0.firebaseio.com/"+email[0],"/fingerprintImage")

#print('Finger enrolled successfully!')

#pass

else:

print("Your email id is not matching with our databse Please try agian later time")

#positionNumber = input('Please enter the template position you want to delete: ')

positionNumber = int(positionNumber)

if ( f.deleteTemplate(positionNumber) == True ):

print('Fingerprint has been deleted from our database!!!')

firebase.delete("https://qrmoney-1aec0.firebaseio.com/"+email[0],"/fingerprintImage")

#except Exception as e:

# print('Operation failed!')

#print('Exception message: ' + str(e))

#exit(1)

try:

label = Label(root, text="Please enter your email: ")

label.grid(row = 0, column = 1)

entry = Entry()

entry.grid(row = 1, column = 1)

label3 = Label(root)

label3.grid(row=2, column = 0)

enroll1 = Button(text="Enroll", command=enroll).grid(row = 4, column = 0)

search = Button(text="Search", command=search).grid(row = 4, column = 1)

delete = Button(text="Delete", command=delete).grid(row = 4, column = 2)

#txt = Text(root, width=45, height=15)

txt.grid(row = 6, column = 0)

#txt2 = Text(root, width=45, height=15)

txt2.grid(row = 6, column = 1)

txt3.grid(row = 6, column = 2)

root.mainloop()

except Exception as e:

print('Operation failed!')

print('Exception message: ' + str(e))

exit(1)

11.4 Android java code Contact.java

package com.example.susha.qrmoney;
import android.widget.EditText;
/**
 * Created by susha on 2018-03-02.
 */
public class contact {
    String acbal;
    String cardNumber, email, password;

    public contact(EditText cardNumber, EditText email, EditText password) {
    }
    public contact(String cardNo, String Password) {
        cardNumber = cardNo;
        password = Password;
       // email = finalEmail;
    }

    public void setAcbal(String acbal){

        this.acbal = acbal;
    }
    public String getaccbal (){

        return this.acbal;
    }
    public void setCardNumber(String cardNumber){
        this.cardNumber = cardNumber;
    }

    public String getCardNumber(){
        return this.cardNumber;
    }

    public void setEmail(String email){
        this.email = email;
    }
    public String getEmail(){
        return this.email;
    }

    public void setPassword(String password){
        this.password = password;
    }

    public String getPassword(){
        return this.password;
    }
}

11.5 Android java code db.java

package com.example.susha.qrmoney;
/**
 * Created by vino on 2018-04-01.
 */
public class db {
    String accountnumber = null;
    String accountbalance = null;

public db(String accountbalance, String accountnumber){
        this.accountbalance = accountbalance;
        this.accountnumber = accountnumber;
}
public void setAccountnumber(String accountnumber){
        this.accountnumber = accountnumber;
    }

    public void setAccountbalance(String accountbalance) {
        this.accountbalance = accountbalance;
    }
    public String getAccountnumber(){
    return this.accountnumber;

    }
    public String getAccountbalance(){

        return this.accountbalance;
    }
}

11.6 Android java code Default.java

package com.example.susha.qrmoney;

import android.annotation.SuppressLint;
import android.app.Activity;
import android.app.AlertDialog;
import android.app.FragmentManager;
import android.content.Context;
import android.content.DialogInterface;
import android.content.Intent;
import android.support.annotation.NonNull;
import android.support.annotation.Nullable;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentActivity;
import android.support.v7.app.AppCompatActivity;
import android.os.Bundle;
import android.support.v7.widget.CardView;
import android.text.InputFilter;
import android.text.Layout;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.FrameLayout;

import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;
import com.google.firebase.database.DatabaseReference;
import com.google.firebase.database.FirebaseDatabase;

import static java.security.AccessController.getContext;

public class Default extends AppCompatActivity implements View.OnClickListener {


    static CardView logincardview;
    static CardView enrolcardview;
    static CardView paycardview;
    static CardView transfercardview;
    static CardView parkcardview;
    static Default activityA;
    private FirebaseAuth mAuth;
    FirebaseUser currentUser;
    View v;

    public Default(CardView logincardview, CardView enrolcardview, CardView paycardview, CardView transfercardview, CardView parkcardview) {
        this.logincardview = logincardview;
        this.enrolcardview = enrolcardview;
        this.parkcardview = parkcardview;
        this.paycardview = paycardview;
        this.transfercardview = transfercardview;
    }

    public Default() {}



    @Override
    protected void onCreate(final Bundle savedInstanceState) {
        super.onCreate(savedInstanceState);
        setContentView(R.layout.activity_default);
        activityA = this;
        FirebaseDatabase database;
        final DatabaseReference users;
        mAuth = FirebaseAuth.getInstance();
        currentUser = mAuth.getCurrentUser();
        logincardview = (CardView) findViewById(R.id.Logindefualt);
        enrolcardview = (CardView) findViewById(R.id.enroldefault);
        paycardview = (CardView) findViewById(R.id.paydefault);
        transfercardview = (CardView) findViewById(R.id.transferdefault);
        parkcardview = (CardView) findViewById(R.id.parkingdefault);
        boolean clicked = false;
        mAuth = FirebaseAuth.getInstance();
        final FirebaseUser currentUser = mAuth.getCurrentUser();
         final MainActivity mainActivity = new MainActivity();
       logincardview.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {

                Intent i = new Intent(Default.this, MainActivity.class);
                 Default.this.startActivity(i);

            }
        });

        enrolcardview.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {

                //Intent f = new Intent(Default.this, MainActivity.class);
                //Default.this.startActivity(f);
                //MainActivity activity = (MainActivity) getActivity();
                //MainActivity mainActivity1 = new MainActivity();
               // mainActivity.updateUI(currentUser);

                   int CONTENT_VIEW_ID = 10101010;
                    FrameLayout frame = new FrameLayout(Default.this);
                //Layout frame = new R.id.fragmentcontainer;

                    frame.setId(CONTENT_VIEW_ID);
                    setContentView(frame, new FrameLayout.LayoutParams(
                            FrameLayout.LayoutParams.MATCH_PARENT, FrameLayout.LayoutParams.MATCH_PARENT));
                    Fragment fragment = null;
                    fragment = new Enrolfragment();

                    if (fragment != null) {
                        android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
                        android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                        ft.replace(CONTENT_VIEW_ID, fragment);
                        ft.commit();
                    }
                else{

                    final AlertDialog.Builder alert = new AlertDialog.Builder(Default.this);
                    alert.setMessage(" you are currently loged in");
                    alert.setTitle("Pay Alert");

                    alert.setNegativeButton("Cancel", new DialogInterface.OnClickListener() {
                        public void onClick(DialogInterface dialog, int id) {
                            dialog.cancel();
                        }
                    });
                    alert.show();
              }

            }
        });

        paycardview.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                if (currentUser != null) {

                    int CONTENT_VIEW_ID = 1011111;
                    FrameLayout frame = new FrameLayout(Default.this);


                    frame.setId(CONTENT_VIEW_ID);
                    setContentView(frame, new FrameLayout.LayoutParams(
                            FrameLayout.LayoutParams.MATCH_PARENT, FrameLayout.LayoutParams.MATCH_PARENT));
                    Fragment fragment = null;
                    fragment = new pay();

                    if (fragment != null) {
                        android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
                        android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                        ft.replace(CONTENT_VIEW_ID, fragment);
                        ft.commit();
                    }

               }
                else{
                    final AlertDialog.Builder alert = new AlertDialog.Builder(Default.this);
                    alert.setMessage("In order use this function, You Must Login First ");
                    alert.setTitle("Pay Alert");

                    alert.setNegativeButton("Cancel", new DialogInterface.OnClickListener() {
                        public void onClick(DialogInterface dialog, int id) {
                            dialog.cancel();
                        }
                    });
                    alert.show();

                }
            }

        });

        transfercardview.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                if (currentUser == null) {

                    final AlertDialog.Builder alert = new AlertDialog.Builder(Default.this);
                    alert.setMessage("In order use this function, You Must Login First ");
                    alert.setTitle("Transfer");
                    alert.setNegativeButton("Cancel", new DialogInterface.OnClickListener() {
                        public void onClick(DialogInterface dialog, int id) {
                            dialog.cancel();
                        }
                    });
                    alert.show();

               }else{
                int CONTENT_VIEW_ID = 12020202;
                FrameLayout frame = new FrameLayout(Default.this);


                frame.setId(CONTENT_VIEW_ID);
                setContentView(frame, new FrameLayout.LayoutParams(
                        FrameLayout.LayoutParams.MATCH_PARENT, FrameLayout.LayoutParams.MATCH_PARENT));
                Fragment fragment = null;
                    fragment = new transfer();

                if (fragment != null) {
                    android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
                    android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                    ft.replace(CONTENT_VIEW_ID, fragment);
                    ft.commit();
                }
                }

            }
        });
        parkcardview.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                final AlertDialog.Builder alert = new AlertDialog.Builder(Default.this);
                alert.setMessage("This Service is not available for use yet ");
                alert.setTitle("Pay Parking");
                alert.setNegativeButton("Cancel", new DialogInterface.OnClickListener() {
                    public void onClick(DialogInterface dialog, int id) {
                        dialog.cancel();
                    }
                });
                alert.show();
            }

        });

    }
    @Override
    public void onClick(View v) {

    }}

11.7 Android java code Email.java

package com.example.susha.qrmoney;

import android.app.ProgressDialog;
import android.content.Context;
import android.graphics.Bitmap;
import android.graphics.ImageFormat;
import android.media.Image;
import android.os.AsyncTask;
import android.widget.ImageView;
import android.widget.Toast;

import java.io.File;
import java.util.Properties;

import javax.activation.DataHandler;
import javax.activation.DataSource;
import javax.activation.FileDataSource;
import javax.mail.Authenticator;
import javax.mail.BodyPart;
import javax.mail.Message;
import javax.mail.MessagingException;
import javax.mail.Multipart;
import javax.mail.PasswordAuthentication;
import javax.mail.Session;
import javax.mail.Transport;
import javax.mail.internet.InternetAddress;
import javax.mail.internet.MimeBodyPart;
import javax.mail.internet.MimeMessage;
import javax.mail.internet.MimeMultipart;

import static android.R.attr.bitmap;
import static android.R.id.message;
//import static com.example.vino.project.R.id.image;
//import static com.example.vino.project.R.id.img;

/**
 * Created by vino on 2017-08-05.
 */

public class Email extends AsyncTask <Void, Void, Void> {

    private Context context;
    private Session session;
    String email;
    private ProgressDialog progressDialog;
    //MainActivity m = new MainActivity(this.)

    MainActivity main = new MainActivity();
    //File myDirtest = new File();


    public Email (Context context, String email){
        this.context = context;
        this.email = email;




    }


    @Override
    protected Void doInBackground(Void... params) {

        //main.SaveImage(bitmap);
        //File mydirtest = new File(String.valueOf(main.myDir));

        Properties properties = new Properties();
        properties.put("mail.smtp.host", "smtp.gmail.com");
        properties.put("mail.smtp.socketFactory.port", "465");
        properties.put("mail.smtp.socketFactory.class", "javax.net.ssl.SSLSocketFactory");
        properties.put("mail.smtp.port", "465");
        properties.put("mail.smtp.auth", "true");


        session = Session.getDefaultInstance(properties, new Authenticator() {

            protected PasswordAuthentication getPasswordAuthentication() {

                return new PasswordAuthentication(email_auth.EMAIL, email_auth.PASSWORD);


            }

        });

        try {

            // Multipart multipart = new MimeMultipart();
            MimeMessage mimeMessage = new MimeMessage(session);
            mimeMessage.setFrom(new InternetAddress(email_auth.EMAIL));
            mimeMessage.addRecipient(Message.RecipientType.TO, new InternetAddress(email));
            MimeBodyPart messageBodyPart = new MimeBodyPart();
            Multipart multipart = new MimeMultipart();
            messageBodyPart = new MimeBodyPart();
            String file = "/sdcard/Pictures/Image.png";
            String fileName = "Image.png";
            DataSource source = new FileDataSource(file);
            messageBodyPart.setDataHandler(new DataHandler(source));
            messageBodyPart.setFileName(fileName);
            multipart.addBodyPart(messageBodyPart);
            mimeMessage.setContent(multipart);
            mimeMessage.setSubject("Your eMoney is here, please use scanner to scan it.");
            Transport.send(mimeMessage);

        } catch (MessagingException e) {

            e.printStackTrace();
        }
        catch (Exception e){
            e.printStackTrace();
        }


        return null;
    }

    @Override
    protected void onPostExecute(Void aVoid) {
        super.onPostExecute(aVoid);
        progressDialog.dismiss();
        Toast.makeText(context, "Email Sent", Toast.LENGTH_LONG).show();


    }

    @Override
    protected void onPreExecute() {
        super.onPreExecute();
        progressDialog = ProgressDialog.show(context,"Sending Email", "Please Wait....", false,false);
    }
}

11.8 Android java code email_auth.java

package com.example.susha.qrmoney;

/**
 * Created by susha on 2018-03-04.
 */

public class email_auth {

    public static final String EMAIL = "qr.donotreply@gmail.com";
    public static final  String PASSWORD = "humber123";
}

11.9 Android java code Enrolfragment.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.Context;
import android.content.DialogInterface;
import android.content.Intent;
import android.net.Uri;
import android.os.Bundle;
import android.support.annotation.FloatRange;
import android.support.annotation.NonNull;
import android.support.annotation.Nullable;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentManager;
import android.support.v4.app.FragmentTransaction;
import android.support.v7.app.AppCompatActivity;
import android.text.TextUtils;
import android.util.Log;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.EditText;
import android.widget.Toast;

import com.google.android.gms.tasks.OnCompleteListener;
import com.google.android.gms.tasks.Task;
import com.google.firebase.auth.AuthResult;
import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;
import com.google.firebase.database.DataSnapshot;
import com.google.firebase.database.DatabaseError;
import com.google.firebase.database.DatabaseReference;
import com.google.firebase.database.FirebaseDatabase;
import com.google.firebase.database.ServerValue;
import com.google.firebase.database.ValueEventListener;

import java.text.DateFormat;
import java.util.Date;
import java.util.HashMap;
import java.util.Map;
import java.util.regex.Pattern;

import static android.content.ContentValues.TAG;
import static java.text.DateFormat.getDateTimeInstance;


public class Enrolfragment extends Fragment {
    EditText cardNumber, Email, Password, Confirmpassword;
    Button Enrol;
    Button Login;
    private FirebaseAuth mAuth;
    Fragment fragment;
    Fragment frag;
    String emailfinal, passwordfinal, cardnumberfinal;
    int accountbalancefinal;
    DatabaseReference myRef;
    FirebaseDatabase database;
    String Emailstr, passwordstr, confirmpassstr, emailPatten;
    String cardstr;
    Button loginenrol;


    @Override
    public View onCreateView(LayoutInflater inflater, ViewGroup container,
                             Bundle savedInstanceState) {
        View view = inflater.inflate(R.layout.fragment_enrolfragment, null);
        cardNumber = (EditText) view.findViewById(R.id.cardtext);
        Email = (EditText) view.findViewById(R.id.emailtxt);
        Password = (EditText) view.findViewById(R.id.passtxt);
        Confirmpassword = (EditText) view.findViewById(R.id.conpasstxt);
        confirmpassstr = Confirmpassword.getText().toString().trim();
        mAuth = FirebaseAuth.getInstance();
        emailPatten = "[a-zA-Z0-9._-]+@[a-z]+\\.+[a-z]+";
        Button Enrol = (Button) view.findViewById(R.id.enrolbtn);
        loginenrol = (Button) view.findViewById(R.id.loginbtn);
        Button cancelenrol = (Button) view.findViewById(R.id.cancelbtn);
        //ref.child("yourNode").updateChildren(map);

        Enrol.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                createAccount(Email.getText().toString(),
                        Password.getText().toString());


            }
        });
      loginenrol.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                logn();
            }


       });


        cancelenrol.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                Enrolfragment.this.onDetach();

            }
        });
        return view;
    }

    ///Writing to database
    public void writeNewUser(String cardstr, String emails, String password, int accountbalance) {
        int balance = 0;
        contact user = new contact(cardNumber, Email, Password);
        cardstr = cardNumber.getText().toString().trim();
        emails = Email.getText().toString().trim();
        password = Password.getText().toString().trim();
        database = FirebaseDatabase.getInstance();
        String[] parts = emails.split(Pattern.quote("."));
       Map map = new HashMap();
        FirebaseUser user1;
        myRef = database.getReference(parts[0]);
        DatabaseReference usersRef = myRef.child("Email");
        DatabaseReference usersRef1 = myRef.child("CardNumber");
        DatabaseReference usersRef2 = myRef.child("Password");
        DatabaseReference usersRef3 = myRef.child("Account Balance");
       DatabaseReference userRef4 = myRef.child("time");
        usersRef.setValue(emails);
        usersRef1.setValue(cardstr);
        usersRef2.setValue(password);
        usersRef3.setValue(balance);
      userRef4.setValue(ServerValue.TIMESTAMP);

    }

   public Fragment logn() {
        fragment = new Loginfragment();
        return  fragment;
    }

    /*public static String getTimeDate(long timeStamp){
        try{
            DateFormat dateFormat = getDateTimeInstance();
            Date netDate = (new Date(timeStamp));
            return dateFormat.format(netDate);
        } catch(Exception e) {
            return "date";
        }
    }*/


    @Override
    public void onViewCreated(View view, @Nullable Bundle savedInstanceState) {
        super.onViewCreated(view, savedInstanceState);}



    @Override
    public void onStart() {
        super.onStart();
        // Check if user is signed in (non-null) and update UI accordingly.
       // FirebaseUser currentUser = mAuth.getCurrentUser();
        //updateUI(currentUser);
    }


    private void createAccount(String email, String password) {
        if (!validateForm()) {
            return;
        }
        mAuth.createUserWithEmailAndPassword(email, password)
                .addOnCompleteListener(Enrolfragment.this.getActivity(), new OnCompleteListener<AuthResult>() {
                    @Override
                    public void onComplete(@NonNull Task<AuthResult> task) {
                        if (task.isSuccessful()) {
                            writeNewUser(emailfinal, passwordfinal, cardnumberfinal, accountbalancefinal);

                                AlertDialog.Builder alert = new AlertDialog.Builder(getContext());
                            alert.setMessage("You have Registered successfully, Please Click either Login or Cancel ");
                            alert.setTitle("Registration successful");
                            alert.setNeutralButton("go Back", new DialogInterface.OnClickListener() {
                                @Override
                                public void onClick(DialogInterface dialog, int which) {
                                    fragment = new Loginfragment();


                            }

                            });

                            alert.show();

                        } else {
                            // If sign in fails, display a message to the user.
                            Toast.makeText(Enrolfragment.this.getActivity(), "Authentication failed.",
                                    Toast.LENGTH_SHORT).show();
                            updateUI(null);

                        }
                    }
                });
    }

    private boolean validateForm() {
        boolean valid = true;

        Emailstr = Email.getText().toString();
        passwordstr = Password.getText().toString();
        cardstr = cardNumber.getText().toString().trim();
        confirmpassstr = Confirmpassword.getText().toString().trim();
        String emailPatten = "[a-zA-Z0-9._-]+@[a-z]+\\.+[a-z]+";

        if (TextUtils.isEmpty(Emailstr)) {
            Email.setError("Required.");
            valid = false;
        } else {
            Email.setError(null);
        }
        if (!Emailstr.matches(emailPatten)) {
            Email.setError("Email is not vaild");
            valid = false;
        }else {
            Email.setError(null);
        }

        if (cardstr.length() < 10){
            cardNumber.setError("CardNumber isn't valid");
        }else {
            cardNumber.setError(null);
        }

        if (TextUtils.isEmpty(passwordstr)) {
            Password.setError("Required.");
            valid = false;
        } else {
            Password.setError(null);
        }

        if (!passwordstr.equals(confirmpassstr)) {
            Password.setError("Password do not match");
            valid = false;
        }else {
            Password.setError(null);
        }
            return valid;
        }


    private void updateUI(FirebaseUser user) {
        if (user != null) {

        } else {
        }
    }
}

11.9 Android java code Loginfragment.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.Context;
import android.content.DialogInterface;
import android.content.Intent;
import android.content.SharedPreferences;
import android.os.Bundle;
import com.example.susha.qrmoney.db;
import android.preference.PreferenceManager;
import android.support.annotation.NonNull;
import android.support.design.widget.NavigationView;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentManager;
import android.support.v4.app.FragmentTransaction;
import android.support.v7.app.AppCompatActivity;
import android.text.TextUtils;
import android.util.AttributeSet;
import android.util.Log;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.EditText;
import android.widget.FrameLayout;
import android.widget.ProgressBar;
import android.widget.Switch;
import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;

import android.widget.TextView;
import android.widget.Toast;
import com.google.android.gms.tasks.OnCompleteListener;
import com.google.android.gms.tasks.Task;
import com.google.firebase.auth.AuthResult;
import com.google.firebase.database.DataSnapshot;
import com.google.firebase.database.DatabaseError;
import com.google.firebase.database.DatabaseReference;
import com.google.firebase.database.FirebaseDatabase;
import com.google.firebase.database.ValueEventListener;

import com.example.susha.qrmoney.Model.User;
import com.example.susha.qrmoney.Overview;


import java.sql.Array;
import java.util.ArrayList;
import java.util.regex.Pattern;
import static android.content.ContentValues.TAG;


public class Loginfragment extends Fragment {

    FirebaseDatabase database;
    DatabaseReference users;
    ArrayList<db> vino;
    EditText cardNumber, Email, editPassword;
    Button Enrol, Login;
    private Switch savepass;
    Fragment fragment;
    NavigationView navigationView;
    DataSnapshot dataSnapshot;
    String msg = "Sign in error";
    ProgressBar progressBar;
    String cardNo, us;
    String password, cardno, ps;
    // int cardno;
    String customToken;
    Enrolfragment enrolfragment = new Enrolfragment();
    private FirebaseAuth mAuth;
    String accountbal;
    String accountno;
    TextView displaycard, displaybal;
    int acc;
    private String mCustomToken;
    ArrayList<User> val;

    SharedPreferences prefs;
    SharedPreferences.Editor editor;


    String uid = "welcometohumber";

public Loginfragment(){}

    @Override
    public View onCreateView(LayoutInflater inflater, ViewGroup container,
                             Bundle savedInstanceState) {
        View view = inflater.inflate(R.layout.fragment_loginfragment, container, false);
        database = FirebaseDatabase.getInstance();
        editPassword = (EditText) view.findViewById(R.id.passtxt1);
        Enrol = (Button) view.findViewById(R.id.enrolbtn1);
        Login = (Button) view.findViewById(R.id.loginbtn1);
        savepass = (Switch) view.findViewById(R.id.savepass);
        Email = (EditText) view.findViewById(R.id.loginemail);
        mAuth = FirebaseAuth.getInstance();
        //displaybal = (TextView)view.findViewById(R.id.displaybal);
        //displaycard = (TextView)view.findViewById(R.id.displaynumber);

        prefs = getActivity().getSharedPreferences("pref", Context.MODE_PRIVATE);
        editor = prefs.edit();

        Login.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View view) {
                signIn(
                        Email.getText().toString(),
                        editPassword.getText().toString()
                );
            }
        });


        Enrol.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                fragment = new Enrolfragment();

                if (fragment != null) {
                    android.support.v4.app.FragmentManager fragmentManager = getFragmentManager();
                    android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                    ft.replace(R.id.fragmentcontainer, fragment);
                    ft.commit();
                }

            }
        });
        return view;
    }

    @Override
    public void onStart() {

        FirebaseUser currentUser = mAuth.getCurrentUser();

            super.onStart();

    }
    public void updateUI(FirebaseUser user) {
        //FirebaseUser cu = mAuth.getCurrentUser();
        if (user != null) {
            Fragment fragment = new Overview();
            android.support.v4.app.FragmentManager fragmentManager = getFragmentManager();
            android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
            ft.replace(R.id.fragmentcontainer, fragment);
            ft.commit();
        }
        else{

        }

    }

    public boolean validateForm() {
        boolean valid = true;

        //String email = mEmailField.getText().toString();
        String email = Email.getText().toString();
        if (TextUtils.isEmpty(email)) {
            Email.setError("Required.");
            valid = false;
        } else {
            Email.setError(null);
        }

        String password = editPassword.getText().toString();
        if (TextUtils.isEmpty(password)) {
            editPassword.setError("Required.");
            valid = false;
        } else {
            editPassword.setError(null);
        }

        return valid;
    }


    public void signIn(final String email, String password) {

       // Log.d(TAG, "signIn:" + email);
        Toast.makeText(Loginfragment.this.getActivity(), "sigIn" + email,
                Toast.LENGTH_SHORT).show();
        if (!validateForm()) {
            return;
        }

        // [START sign_in_with_email]
        mAuth.signInWithEmailAndPassword(email, password)
                .addOnCompleteListener(Loginfragment.this.getActivity(), new OnCompleteListener<AuthResult>() {
                    @Override
                    public void onComplete(@NonNull Task<AuthResult> task) {
                        if (task.isSuccessful()) {
                            Toast.makeText(Loginfragment.this.getActivity(), "Successfully Logged in!!",
                                    Toast.LENGTH_SHORT).show();
                            final FirebaseUser user = mAuth.getCurrentUser();
                            if (user != null) {
                                //Overview overview  = new Overview();
                                // Fragment f = new Loginfragment()
                                // fragment = new Overview();
                                //updateUI(user);

                                //mainActivity.fragment2
                                //android.support.v4.app.FragmentManager fragmentManager = getFragmentManager();
                                //android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                                //ft.replace(R.id.fragmentcontainer, fragment);
                                //ft.commit();
                                Intent i = new Intent(Loginfragment.this.getActivity(), MainActivity.class);
                                Loginfragment.this.startActivity(i);
                            }
                            else{
                                //
                            }

                        } else {
                            // If sign in fails, display a message to the user.
                            Toast.makeText(Loginfragment.this.getActivity(), "Authentication failed.",
                                    Toast.LENGTH_SHORT).show();
                            updateUI(null);
                        }

                        // [START_EXCLUDE]
                        if (!task.isSuccessful()) {
                            //mStatusTextView.setText(R.string.auth_failed);
                            Toast.makeText(Loginfragment.this.getActivity(), "Authentication failed.",
                                    Toast.LENGTH_SHORT).show();
                        }
                    }
                });
    }

    public void bringdata(String accountbalance, String accountnu) {
        User user = new User(cardno, accountbal);
        users.addValueEventListener(new ValueEventListener() {
            @Override
            public void onDataChange(DataSnapshot dataSnapshot) {
                cardno = dataSnapshot.child("CardNumber").getValue().toString();
                accountbal = dataSnapshot.child("Account Balanace").getValue().toString();
            }
            @Override
            public void onCancelled(DatabaseError databaseError) {

            }
        });

    }
}

11.10 Android java code MainActivity.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.DialogInterface;
import android.content.Intent;
import android.os.Bundle;
import android.support.design.widget.NavigationView;
import android.support.transition.Slide;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentManager;
import android.support.v4.view.GravityCompat;
import android.support.v4.widget.DrawerLayout;
import android.support.v7.app.ActionBarDrawerToggle;
import android.support.v7.app.AppCompatActivity;
import android.support.v7.widget.CardView;
import android.support.v7.widget.Toolbar;
import android.view.Gravity;
import android.view.Menu;
import android.view.MenuItem;
import android.view.View;
import android.widget.Button;
import android.widget.TextView;

import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;
import com.google.firebase.database.DataSnapshot;
import com.google.firebase.database.DatabaseError;
import com.google.firebase.database.DatabaseReference;
import com.google.firebase.database.FirebaseDatabase;
import com.google.firebase.database.ValueEventListener;

import java.util.regex.Pattern;

import static com.example.susha.qrmoney.Default.activityA;
import static com.example.susha.qrmoney.Default.enrolcardview;
import static com.example.susha.qrmoney.Overview.displaybal;
import static com.example.susha.qrmoney.Overview.displaycard;


public class MainActivity extends AppCompatActivity

        implements NavigationView.OnNavigationItemSelectedListener {
  private FirebaseAuth mAuth;
   FirebaseUser currentUser;
  FirebaseDatabase database;
   DatabaseReference users;
    Fragment fragment2 = null;
  String test;
  String test2;
  String test3;
  String c, d;
  //boolean click = false;
    Fragment fragment = null;
    Button loginbtnmain, enrolmain, paybtnmain, transferbtnmain;
    Loginfragment loginfragment = new Loginfragment();
    Default aDefault = new Default();

    @Override
    protected void onCreate(Bundle savedInstanceState) {
        super.onCreate(savedInstanceState);
        setContentView(R.layout.activity_main);
        Toolbar toolbar = (Toolbar) findViewById(R.id.toolbar);
        setSupportActionBar(toolbar);

        loginbtnmain = (Button) findViewById(R.id.Logindefualt);
        enrolmain = (Button) findViewById(R.id.enroldefault);
        paybtnmain = (Button) findViewById(R.id.paydefault);
        transferbtnmain = (Button) findViewById(R.id.transferdefault);

        final DrawerLayout drawer = (DrawerLayout) findViewById(R.id.drawer_layout);
        ActionBarDrawerToggle toggle = new ActionBarDrawerToggle(
                this, drawer, toolbar, R.string.navigation_drawer_open, R.string.navigation_drawer_close);
        drawer.addDrawerListener(toggle);
        toggle.syncState();

        View v = null;
        NavigationView navigationView = (NavigationView) findViewById(R.id.nav_view);
        navigationView.setNavigationItemSelectedListener(this);

        mAuth = FirebaseAuth.getInstance();
        final FirebaseUser currentUser = mAuth.getCurrentUser();

        if (currentUser != null) {
            fragment = new Overview();
            DrawerLayout drawer1 = (DrawerLayout) findViewById(R.id.drawer_layout);
            drawer1.closeDrawer(GravityCompat.START);
        }
        else {
            fragment = new Loginfragment();
            //drawer.closeDrawer(GravityCompat.END);
        }



        if (fragment != null) {
            android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
            android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
            ft.replace(R.id.fragmentcontainer, fragment);
            ft.commit();}
    }



    public void updateUI(FirebaseUser user) {
        //FirebaseUser cu = mAuth.getCurrentUser();
        //if (user != null) {

           // Fragment fragment = new Enrolfragment();

           // android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
           // android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
            //ft.replace(R.id.fragmentcontainer, fragment);
            //ft.commit();}

       // else{

       // }

    }

    @Override
    protected void onDestroy() {
        FirebaseAuth.getInstance().signOut();
        super.onDestroy();
    }
    @Override
    public void onBackPressed() {
        DrawerLayout drawer = (DrawerLayout) findViewById(R.id.drawer_layout);
        if (drawer.isDrawerOpen(GravityCompat.START)) {
            drawer.closeDrawer(GravityCompat.START);
        } else {
            super.onBackPressed();
        }
    }

    @Override
    public boolean onCreateOptionsMenu(Menu menu) {
        // Inflate the menu; this adds items to the action bar if it is present.
        getMenuInflater().inflate(R.menu.main, menu);
        return true;
    }

    @Override
    public boolean onOptionsItemSelected(MenuItem item) {
        int id = item.getItemId();
        //noinspection SimplifiableIfStatement
       // if (id == R.id.action_settings) {
            //return true;
        //}
        return super.onOptionsItemSelected(item);
    }


    public void takedata(){
        database = FirebaseDatabase.getInstance();
        mAuth = FirebaseAuth.getInstance();
        FirebaseUser currentUser = mAuth.getCurrentUser();
        if (currentUser != null) {
            String id = currentUser.getEmail();
            //String test = id.getEmail();
            String[] partsfinal = id.split(Pattern.quote("."));
            users = database.getReference(partsfinal[0]);
            users.addValueEventListener(new ValueEventListener() {
                @Override
                public void onDataChange(DataSnapshot dataSnapshot) {
                    test = dataSnapshot.child("CardNumber").getValue().toString();
                    test2 = dataSnapshot.child("Account Balance").getValue().toString();
                    test3 = dataSnapshot.getKey().toString();
                }


                @Override
                public void onCancelled(DatabaseError databaseError) {

                }
            });
        }else{
            //
        }
    }

    @SuppressWarnings("StatementWithEmptyBody")

    @Override
    public boolean onNavigationItemSelected(MenuItem item) {
       //String test = "vino";
        // Handle navigation view item clicks here.
       Fragment fragment1 = new Loginfragment();
        FirebaseUser currentUser = mAuth.getCurrentUser();
        android.support.v4.app.FragmentManager fm = getSupportFragmentManager();
        Fragment fragment = null;
        //Fragment fragment2 = null;
        int id = item.getItemId();

        if (id == R.id.dashboard) {
            fragment = new Overview();

        } else if (id == R.id.paydefault) {
            fragment = new pay();
        } else if (id == R.id.pay) {
            fragment = new pay();

        } else if (id == R.id.transfer) {
            fragment = new transfer();

        } else if (id == R.id.wallet) {

        } else if (id == R.id.logoutbtn){
            final AlertDialog.Builder alert = new AlertDialog.Builder(MainActivity.this);
            alert.setMessage("Do you want to logout? ");
            alert.setTitle("Logout");

            alert.setNeutralButton("No", new DialogInterface.OnClickListener() {
                public void onClick(DialogInterface dialog, int id) {
                    dialog.cancel();
                }
            });
            alert.setNegativeButton("Yes", new DialogInterface.OnClickListener() {
                @Override
                public void onClick(DialogInterface dialog, int which) {
                    FirebaseAuth.getInstance().signOut();
                    Intent i = new Intent(MainActivity.this, Default.class);
                    MainActivity.this.startActivity(i);

                }
            });
            alert.show();
        }

        if (fragment != null) {
            android.support.v4.app.FragmentManager fragmentManager = getSupportFragmentManager();
            android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
            ft.replace(R.id.fragmentcontainer, fragment);
            ft.commit();

        }
        DrawerLayout drawer = (DrawerLayout) findViewById(R.id.drawer_layout);
        drawer.closeDrawer(GravityCompat.START);
        return true;
    }
}

11.11 Android java code Overview.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.Context;
import android.content.DialogInterface;
import android.content.Intent;
import android.net.Uri;
import android.os.Bundle;
import android.support.annotation.NonNull;
import android.support.annotation.Nullable;
import android.support.design.widget.NavigationView;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentManager;
import android.support.v4.app.FragmentTransaction;
import android.util.Log;
import android.view.LayoutInflater;
import android.view.Menu;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.TextView;
import android.widget.Toast;

import com.example.susha.qrmoney.Model.User;
import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;
import com.google.firebase.database.DataSnapshot;
import com.google.firebase.database.DatabaseError;
import com.google.firebase.database.DatabaseReference;
import com.google.firebase.database.FirebaseDatabase;
import com.google.firebase.database.Query;
import com.google.firebase.database.ValueEventListener;
import  com.example.susha.qrmoney.Loginfragment;

import java.util.ArrayList;
import java.util.List;
import java.util.Map;
import java.util.regex.Pattern;
import com.example.susha.qrmoney.pay;

import static android.content.ContentValues.TAG;


public class Overview extends Fragment  {

Button logoutbtn, cancelbtn;
    Fragment fragment;
    NavigationView navigationView;
   static TextView displaycard, displaybal;
    FirebaseDatabase database;
    FragmentManager fm;
    Loginfragment fragm;
    DatabaseReference users;
    FirebaseAuth mAuth;
    FirebaseUser user;
    String cardno;
    String accountbal;
    Bundle bundle;
    String test, test2, c,a;

    public Overview(){}

    @Override
    public View onCreateView(LayoutInflater inflater, ViewGroup container,
                             Bundle savedInstanceState) {
        View view = inflater.inflate(R.layout.fragment_overview, container, false);
        //MainActivity activity = (MainActivity) getActivity();
       // activity.takedata();
        //displaybal.setText(activity.test2);
        //displaycard.setText(activity.test);

        //view.autofill();
        //view.refreshDrawableState();

        logoutbtn = (Button) view.findViewById(R.id.logoutbtn);
        displaybal = (TextView)view.findViewById(R.id.displaybal);
        displaycard = (TextView)view.findViewById(R.id.displaynumber);
        fm = getFragmentManager();
        cancelbtn = (Button) view.findViewById(R.id.cancelbtnover);
       // MainActivity activity = (MainActivity) getActivity();
       // activity.takedata();
        //displaybal.setText(activity.test2);
        //displaycard.setText(activity.test);

        logoutbtn.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                final AlertDialog.Builder alert = new AlertDialog.Builder(Overview.this.getActivity());
                alert.setMessage("Do you want to logout? ");
                alert.setTitle("Logout");

                alert.setNeutralButton("No", new DialogInterface.OnClickListener() {
                    public void onClick(DialogInterface dialog, int id) {
                        dialog.cancel();
                    }
                });

               alert.setNegativeButton("Yes", new DialogInterface.OnClickListener() {
                   @Override
                   public void onClick(DialogInterface dialog, int which) {
                       FirebaseAuth.getInstance().signOut();
                       Intent i = new Intent(Overview.this.getActivity(), Default.class);
                       Overview.this.startActivity(i);

                   }
               });
                alert.show();
            }
        });

        cancelbtn.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                Intent in = new Intent(Overview.this.getActivity(), Default.class);
                Overview.this.getActivity().startActivity(in);
            }
        });

            return view;

    }
    @Override
    public void onStart() {

        MainActivity activity = (MainActivity) getActivity();
        activity.takedata();
       displaybal.setText(activity.test2);
       displaycard.setText(activity.test);
       c = displaycard.getText().toString();
        a = displaybal.getText().toString();
       super.onStart();
}
}

11.12 Android java code pay.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.DialogInterface;
import android.content.Intent;
import android.graphics.Bitmap;
import android.os.Bundle;
import android.support.annotation.Nullable;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentManager;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.EditText;
import android.widget.ImageView;
import android.widget.Toast;

import com.google.firebase.auth.FirebaseAuth;
import com.google.firebase.auth.FirebaseUser;
import com.google.firebase.database.DataSnapshot;
import com.google.firebase.database.DatabaseError;
import com.google.firebase.database.ValueEventListener;
import com.google.zxing.BarcodeFormat;
import com.google.zxing.MultiFormatWriter;
import com.google.zxing.WriterException;
import com.google.zxing.common.BitMatrix;
import com.journeyapps.barcodescanner.BarcodeEncoder;
import com.example.susha.qrmoney.Loginfragment;
import com.example.susha.qrmoney.Overview;

import static java.lang.Float.parseFloat;
import static java.lang.Integer.parseInt;


public class pay extends Fragment {
    Button back, paybutton;
    EditText amount, accountnumber;
    ImageView img;
    Fragment fragment;
    FirebaseAuth mAuth;
    String balance1;
    String number;
    Bundle bundle;
    String bal;
    @Override
    public View onCreateView(LayoutInflater inflater, ViewGroup container,
                             Bundle savedInstanceState) {
        View view = inflater.inflate(R.layout.fragment_pay, null);
        back = (Button) view.findViewById(R.id.btn12);
        paybutton = (Button) view.findViewById(R.id.paybtn);
        amount = (EditText) view.findViewById(R.id.paytxt);
        //accountnumber = (EditText) view.findViewById(R.id.accountnumberpay);
        img = (ImageView)view.findViewById(R.id.img);
        mAuth = FirebaseAuth.getInstance();

        back.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View view) {
                fragment = new Overview();
                    android.support.v4.app.FragmentManager fragmentManager = getFragmentManager();
                    android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                    ft.replace(R.id.fragmentcontainer, fragment);
                    ft.commit();}

        });


        return view;
    }
    

    @Override
    public void onViewCreated(View view, @Nullable Bundle savedInstanceState) {
        super.onViewCreated(view, savedInstanceState);
        final String[] balance = new String[1];
        final FirebaseUser currentUser = mAuth.getCurrentUser();

        if (currentUser != null) {
            paybutton.setOnClickListener(new View.OnClickListener() {
                @Override
                public void onClick(View view) {
                   // Loginfragment loginfragment = new Loginfragment();
                    //String text2Qr =  amount.getText().toString().trim();
                    String text2Qr = amount.getText().toString();
                   // String pay = accountnumber.getText().toString();
                    MainActivity activity = (MainActivity) getActivity();
                    activity.takedata();

                    FragmentManager fm = getFragmentManager();

                     if (parseFloat(text2Qr) > parseFloat(activity.test2)) {
                        final AlertDialog.Builder alert = new AlertDialog.Builder(pay.this.getActivity());
                        alert.setMessage("You  do not have an siginfcant amount in your account!  ");
                        alert.setTitle("Error");

                        alert.show();

                    }
                    else {
                        String combined = null;
                        combined = activity.test + "," + text2Qr;


                        MultiFormatWriter multiFormatWriter = new MultiFormatWriter();

                        try {

                            BitMatrix bitMatrix = multiFormatWriter.encode(combined, BarcodeFormat.QR_CODE, 300, 300);
                            BarcodeEncoder barcodeEncoder = new BarcodeEncoder();
                            //Bitmap bitmap = barcodeEncoder.createBitmap(bitMatrix);
                            Bitmap finalBitmap = barcodeEncoder.createBitmap(bitMatrix);

                            img.setImageBitmap(finalBitmap);
                        } catch (WriterException e) {
                            e.printStackTrace();

                        }
                   }
                   if ((parseFloat(text2Qr) < parseFloat(activity.test2))){
                         Float newbalace = parseFloat(activity.test2) - parseFloat(text2Qr);
                         activity.users.child("Account Balance").setValue(newbalace);

                    }

                }
            });

        }
    }
}

11.13 Android java code slideActivity.java

package com.example.susha.qrmoney;

import android.support.annotation.NonNull;
import android.support.annotation.Nullable;
import android.support.v4.app.Fragment;
import android.support.v4.view.ViewPager;

import android.support.v7.app.AppCompatActivity;
import android.os.Bundle;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;

public class slideActivity extends AppCompatActivity {
    private ViewPager viewPa;
    private SlideAdapter myadapter;

    @Override
    protected void onCreate(Bundle savedInstanceState) {
        super.onCreate(savedInstanceState);
        setContentView(R.layout.activity_slide);
        viewPa = findViewById(R.id.viewpager);
        myadapter = new SlideAdapter(this);
        viewPa.setAdapter(myadapter);}
}

11.4 Android java code SlideAdapter.java

package com.example.susha.qrmoney;

import android.content.ClipData;
import android.content.Context;
import android.content.Intent;
import android.graphics.Color;
import android.support.annotation.NonNull;
import android.support.v4.app.Fragment;
import android.support.v4.app.FragmentActivity;
import android.support.v4.view.PagerAdapter;
import android.support.v4.widget.DrawerLayout;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.ImageView;
import android.widget.LinearLayout;
import android.widget.TextView;
import com.example.susha.qrmoney.R;

/**
 * Created by vino on 2018-03-21.
 */

public class SlideAdapter extends PagerAdapter{


    Context context;
    LayoutInflater inflater;
    Button btnstart;


    public  int[] lst_image = {
            R.drawable.image_1,
            R.drawable.image_2,
            R.drawable.image_3,
            R.drawable.image_4
    };

    public String[] lst_title = {
            "BANKING",
            "PAY",
            "TRANSFER",
            "PAY PARKING"

    };

    public String[] lst_description = {

            "Banking with QIK pay is simple, all you need is your phone. No cards, No cash",
            "With QiK Pay feature, you can make a payment without the use of card OR cash. Just generate the QR and let them scan it. Done, it's that easy",
            "ansfer money to bank the with Qr code. Eliminate the use of Cards. It's easy with QR QIK Pay",
            "In near Future, QIK pay will take over All the Parking Lots"
    };

    public  int[] lst_backgroundcolor = {
            Color.rgb(55,55,55),
            Color.rgb(239,85,85),
            Color.rgb(110,40,89),
            Color.rgb(1,188,212),};

public SlideAdapter(Context context){
    this.context = context;
}
    @Override
    public int getCount() {
        return lst_title.length;
    }

    @Override
    public boolean isViewFromObject(@NonNull View view, @NonNull Object object) {
        return (view==(LinearLayout)object);}

    @NonNull
    @Override
    public Object instantiateItem(@NonNull ViewGroup container, int position) {
        inflater = (LayoutInflater) context.getSystemService(context.LAYOUT_INFLATER_SERVICE);
        View view = inflater.inflate(R.layout.slide,container,false);
        LinearLayout layoutslide = view.findViewById(R.id.slidelinearlayout);
        ImageView imgslide = (ImageView) view.findViewById(R.id.slideimg);
        TextView txtitle = (TextView) view.findViewById(R.id.txttitle);
        TextView txtdescription = (TextView) view.findViewById(R.id.txtdescription);
       btnstart = (Button) view.findViewById(R.id.btnstart);
       btnstart.setVisibility(View.INVISIBLE);
        if (position == 3) {
            btnstart.setVisibility(View.VISIBLE);

            btnstart.setOnClickListener(new View.OnClickListener() {
                @Override
                public void onClick(View v) {
                    Intent in = new Intent(SlideAdapter.this.context, Default.class);
                    SlideAdapter.this.context.startActivity(in);

                }
            });
        }

        layoutslide.setBackgroundColor(lst_backgroundcolor[position]);
        imgslide.setImageResource(lst_image[position]);
        txtitle.setText(lst_title[position]);
        txtdescription.setText(lst_description[position]);
        container.addView(view);
        return view;

    }

    @Override
    public void destroyItem(@NonNull ViewGroup container, int position, @NonNull Object object) {
       container.removeView((LinearLayout)object);}
}

11.14 Android java code transfer.java

package com.example.susha.qrmoney;

import android.app.AlertDialog;
import android.content.Context;
import android.graphics.Bitmap;
import android.net.Uri;
import android.os.Bundle;
import android.os.Environment;
import android.support.annotation.Nullable;
import android.support.v4.app.Fragment;
import android.view.LayoutInflater;
import android.view.View;
import android.view.ViewGroup;
import android.widget.Button;
import android.widget.EditText;
import android.widget.ImageView;

import com.google.zxing.BarcodeFormat;
import com.google.zxing.MultiFormatWriter;
import com.google.zxing.WriterException;
import com.google.zxing.common.BitMatrix;
import com.journeyapps.barcodescanner.BarcodeEncoder;

import java.io.File;
import java.io.FileOutputStream;
import java.util.Random;
import java.util.regex.Pattern;

import static java.lang.Float.parseFloat;
import static java.lang.Integer.parseInt;


public class transfer extends Fragment {
    Button back, transfer, generate;
    EditText to, subject, amount_txt, who;
    ImageView transferiamge;
    Fragment fragment;
    ImageView img;
    String email = "";
    String subject1 = "";
    Bitmap finalBitmap, map;
    String root = Environment.getExternalStorageDirectory().toString();
    @Override
    public View onCreateView(LayoutInflater inflater, ViewGroup container,
                             Bundle savedInstanceState) {
        // Inflate the layout for this fragment
        View view = inflater.inflate(R.layout.fragment_transfer, container, false);
        back = (Button) view.findViewById(R.id.btn13);
        transfer = (Button)view.findViewById(R.id.transferbtn);
        to = (EditText) view.findViewById(R.id.totxt);
        //who = (EditText) view.findViewById(R.id.);
        //who= (EditText) view.findViewById(R.id.subjecttxt);
        transferiamge = (ImageView) view.findViewById(R.id.transferimage);
        generate = (Button) view.findViewById(R.id.gnbtn);
        amount_txt = (EditText) view.findViewById(R.id.mini);

        return view;
    }


    @Override
    public void onViewCreated(View view, @Nullable Bundle savedInstanceState) {
        super.onViewCreated(view, savedInstanceState);

        back.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View view) {

                if (view.getId() == R.id.btn13) {
                    fragment = new Overview();
                }


                if (fragment != null) {
                    android.support.v4.app.FragmentManager fragmentManager = getFragmentManager();
                    android.support.v4.app.FragmentTransaction ft = fragmentManager.beginTransaction();
                    ft.replace(R.id.fragmentcontainer, fragment);
                    ft.commit();

                }
            }
        });


        generate.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View view) {
                MainActivity activity = (MainActivity) getActivity();
                activity.takedata();
                String text2Qr =  amount_txt.getText().toString().trim();
                String pay = to.getText().toString().trim();
                MultiFormatWriter multiFormatWriter = new MultiFormatWriter();
                if (parseFloat(text2Qr) > parseFloat(activity.test2)) {
                    final AlertDialog.Builder alert = new AlertDialog.Builder(transfer.this.getActivity());
                    alert.setMessage("You  do not have an siginfcant amount in your account!  ");
                    alert.setTitle("Error");

                    alert.show();

                }
                else {
                    //String combined = activity.test3 + "," + text2Qr;
                    String[] tofinal = pay.split(Pattern.quote("."));
                    String combined = activity.test3 + "," + tofinal[0] + "," + text2Qr;
                    try {
                        BitMatrix bitMatrix = multiFormatWriter.encode(combined, BarcodeFormat.QR_CODE, 200, 200);
                        BarcodeEncoder barcodeEncoder = new BarcodeEncoder();
                        //Bitmap bitmap = barcodeEncoder.createBitmap(bitMatrix);
                        finalBitmap = barcodeEncoder.createBitmap(bitMatrix);

                        transferiamge.setImageBitmap(finalBitmap);
                        SaveImage(finalBitmap);
                        // show_dialogqr(msgqr);

                    } catch (WriterException e) {
                        e.printStackTrace();

                    }
                }

            }
        });

        transfer.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View view) {
                sendEmail();
            }
        });



    }

    public void transaction(){

    }
    public void sendEmail(){

        email = to.getText().toString().trim(); // trim will remove spaces
        // subject1 = subject.getText().toString().trim();
        //Image Get
        Email m = new Email(getActivity(),email); // this is access the mail file from....
        m.execute(); // this is ecute alll file in asynce

    }
    public void SaveImage(Bitmap finalBitmap) {
        map = finalBitmap;
        ImageView imgtest = transferiamge;
        imgtest.setImageBitmap(finalBitmap);
        File myDir = new File(root + "/Pictures");
        myDir.mkdirs();
        // Random generator = new Random();
        // int n = 10000;
        // n = generator.nextInt(n);
        //String fpath = "/SDCARD/Pictures/";
        //String fname = "Image.png";
        String fname = "Image.png";
        File file = new File(myDir, fname);
        if (file.exists()) file.delete();

        try {
            FileOutputStream out = new FileOutputStream(file);
            // out.write(finalBitmap);

            finalBitmap.compress(Bitmap.CompressFormat.PNG, 90, out);
            out.flush();
            out.close();

        } catch (Exception e) {
            e.printStackTrace();
        }


        //}
    }
}

11.15 Android java code welcomeActivity.java

package com.example.susha.qrmoney;

import android.content.Intent;
import android.support.v7.app.AppCompatActivity;
import android.os.Bundle;
import android.view.View;
import android.view.animation.Animation;
import android.view.animation.AnimationUtils;
import android.widget.Button;
import android.widget.LinearLayout;

public class welcomeActivity extends AppCompatActivity {
    LinearLayout l1,l2;
    Button btnsub;
    Animation uptodown,downtoup;

    @Override
    protected void onCreate(Bundle savedInstanceState) {
        super.onCreate(savedInstanceState);
        setContentView(R.layout.activity_welcome);
        btnsub = (Button)findViewById(R.id.buttonsub);
        l1 = (LinearLayout) findViewById(R.id.l1);
        l2 = (LinearLayout) findViewById(R.id.l2);
        uptodown = AnimationUtils.loadAnimation(this,R.anim.uptodown);
        downtoup = AnimationUtils.loadAnimation(this,R.anim.downtoup);
        l1.setAnimation(uptodown);
        l2.setAnimation(downtoup);

        btnsub.setOnClickListener(new View.OnClickListener() {
            @Override
            public void onClick(View v) {
                Intent intent = new Intent(welcomeActivity.this, slideActivity.class);
               welcomeActivity.this.startActivity(intent);
            }
        });
    }
    }

11.16 Android java code ExampleInstrumentedTest.java

package com.example.susha.qrmoney;

import android.content.Context;
import android.support.test.InstrumentationRegistry;
import android.support.test.runner.AndroidJUnit4;

import org.junit.Test;
import org.junit.runner.RunWith;

import static org.junit.Assert.*;

/**
 * Instrumented test, which will execute on an Android device.
 *
 * @see <a href="http://d.android.com/tools/testing">Testing documentation</a>
 */
@RunWith(AndroidJUnit4.class)
public class ExampleInstrumentedTest {
    @Test
    public void useAppContext() throws Exception {
        // Context of the app under test.
        Context appContext = InstrumentationRegistry.getTargetContext();

        assertEquals("com.example.susha.qrmoney", appContext.getPackageName());
    }
}

11.17 Android java code ExampleUnitTest.java

package com.example.susha.qrmoney;

import org.junit.Test;

import static org.junit.Assert.*;

/**
 * Example local unit test, which will execute on the development machine (host).
 *
 * @see <a href="http://d.android.com/tools/testing">Testing documentation</a>
 */
public class ExampleUnitTest {
    @Test
    public void addition_isCorrect() throws Exception {
        assertEquals(4, 2 + 2);
    }
}

11.18 Android activity_default.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    xmlns:tools="http://schemas.android.com/tools"
    android:orientation="vertical"
    android:padding="10dp"
    android:background="#fcfcfc"
    tools:context=".MainActivity"
    android:gravity="center"
    android:id="@+id/ll">



    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">

        <android.support.v7.widget.CardView
            android:clickable="true"
            android:onClick="onClick"
            android:id="@+id/Logindefualt"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            tools:ignore="OnClick">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/green">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/login_default"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Login" />
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Check your bank activities"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/enroldefault">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/yello">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/signup_defaultf"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Enrol Now"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Enrol now have your bank on your hand"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>

    </LinearLayout>
    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/paydefault">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/lightgray">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/pay_default3"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Pay"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Pay your bills with our app"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/transferdefault">

            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:gravity="center"
                android:orientation="vertical"
                android:background="@color/colorlightcyan">

                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/transfer_default"
                    android:padding="10dp"/>

                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:layout_marginTop="10dp"
                    android:text="Transfer"
                    android:textStyle="bold" />

                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:layout_margin="10dp"
                    android:background="@android:color/black" />

                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:padding="5dp"
                    android:text="Transfer money to your favourite one"
                    android:textColor="@android:color/black" />

            </LinearLayout>
        </android.support.v7.widget.CardView>

    </LinearLayout>
    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="340dp"
            android:layout_height="150dp"
            android:layout_margin="10dp"
            android:id="@+id/parkingdefault">
            <LinearLayout

                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/colororangeplusyellow">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/parking_default"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Pay Park"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Pay your Parking with us"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
    </LinearLayout>
</LinearLayout>

11.19 Android activity_main.xml

<?xml version="1.0" encoding="utf-8"?>
<android.support.v4.widget.DrawerLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:id="@+id/drawer_layout"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    android:fitsSystemWindows="true"
    tools:openDrawer="start">

    <include
        layout="@layout/app_bar_main"
        android:layout_width="match_parent"
        android:layout_height="match_parent"/>

    <android.support.design.widget.NavigationView
        android:id="@+id/nav_view"
        android:layout_width="wrap_content"
        android:layout_height="match_parent"
        android:layout_gravity="start"
        android:fitsSystemWindows="true"
        app:headerLayout="@layout/nav_header_main"
        app:menu="@menu/activity_main_drawer"
        android:background="@color/colorPrimary"/>

</android.support.v4.widget.DrawerLayout>

11.20 Android activity_slide.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    android:orientation="vertical"
    tools:context="com.example.susha.qrmoney.slideActivity">

    <android.support.v4.view.ViewPager
        android:layout_width="match_parent"
        android:layout_height="match_parent"
        android:id="@+id/viewpager">
    </android.support.v4.view.ViewPager>
</LinearLayout>

11.21 Android activity_welcome.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context=".welcomeActivity"
    android:orientation="vertical"
    android:background="@drawable/galaxy3">
    <LinearLayout
        android:id="@+id/l1"
        android:layout_width="match_parent"
        android:layout_height="400dp"
        android:orientation="vertical">

        <TextView
            android:layout_width="wrap_content"
            android:layout_height="wrap_content"
            android:layout_gravity="center"
            android:layout_marginTop="150dp"
            android:text="QIKMONEY"
            android:textColor="@color/colorPrimary"
            android:textSize="60sp"
            android:textStyle="bold" />
        <TextView
            android:textColor="@color/backgroundcolor"
            android:layout_width="398dp"
            android:layout_height="wrap_content"
            android:layout_gravity="center"
            android:text="WE MAKE MONEY HANDLING VERY SIMPLE"
            android:textAlignment="center"
            android:textSize="30sp"
            android:layout_marginTop="50dp"/>
    </LinearLayout>
    <LinearLayout
        android:id="@+id/l2"
        android:layout_marginTop="50dp"
        android:layout_width="match_parent"
        android:layout_height="wrap_content"
        android:orientation="vertical"
        android:background="@drawable/btnimg">
        <Button
            android:id="@+id/buttonsub"
            android:layout_width="wrap_content"
            android:layout_height="wrap_content"
            android:text="Welcome"
            android:layout_gravity="center"
            android:background="@drawable/buttonstyle"
            />

    </LinearLayout>




</LinearLayout>

11.22 Android app_bar_main.xml

<?xml version="1.0" encoding="utf-8"?>
<android.support.design.widget.CoordinatorLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.Default"
    android:theme="@style/AppTheme">
    <android.support.design.widget.AppBarLayout
        android:layout_width="match_parent"
        android:layout_height="wrap_content"
        android:theme="@style/AppTheme.AppBarOverlay">
        <android.support.v7.widget.Toolbar
            android:id="@+id/toolbar"
            android:layout_width="match_parent"
            android:layout_height="?attr/actionBarSize"
            android:background="?attr/colorPrimary"
            app:popupTheme="@style/AppTheme.PopupOverlay" />

    </android.support.design.widget.AppBarLayout>
    <include layout="@layout/content_main" />
</android.support.design.widget.CoordinatorLayout>

11.23 Android content_main.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    app:layout_behavior="@string/appbar_scrolling_view_behavior"
    tools:showIn="@layout/app_bar_main"
    android:id="@+id/fragmentcontainer"
    android:orientation="horizontal">

</LinearLayout>

11.24 Android contenttwo.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    xmlns:tools="http://schemas.android.com/tools"
    android:orientation="vertical"
    android:padding="10dp"
    android:background="#fcfcfc"
    android:gravity="center"
    android:id="@+id/fragmentcontainer1"
    tools:context=".Default">

</LinearLayout>

11.25 Android fragment_defaultfragment.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    android:orientation="vertical"
    android:padding="10dp"
    android:background="#fcfcfc"
    android:gravity="center"
    android:id="@+id/ll">
    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">

        <android.support.v7.widget.CardView
            android:clickable="true"
            android:id="@+id/Logindefualt"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/green">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/login_default"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Login" />
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Check your bank activities"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/enroldefault">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/yello">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/signup_defaultf"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Enrol Now"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Enrol now have your bank on your hand"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>

    </LinearLayout>
    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/paydefault">
            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/lightgray">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/pay_default3"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Pay"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Pay your bills with our app"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="160dp"
            android:layout_height="190dp"
            android:layout_margin="10dp"
            android:id="@+id/transferdefault">

            <LinearLayout
                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:gravity="center"
                android:orientation="vertical"
                android:background="@color/colorlightcyan">

                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/transfer_default"
                    android:padding="10dp"/>

                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:layout_marginTop="10dp"
                    android:text="Transfer"
                    android:textStyle="bold" />

                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:layout_margin="10dp"
                    android:background="@android:color/black" />

                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:padding="5dp"
                    android:text="Transfer money to your favourite one"
                    android:textColor="@android:color/black" />

            </LinearLayout>
        </android.support.v7.widget.CardView>

    </LinearLayout>
    <LinearLayout
        android:clipToPadding="false"
        android:gravity="center"
        android:orientation="horizontal"
        android:layout_width="match_parent"
        android:layout_height="wrap_content">
        <android.support.v7.widget.CardView
            android:foreground="?android:attr/selectableItemBackground"
            android:clickable="true"
            android:layout_width="340dp"
            android:layout_height="150dp"
            android:layout_margin="10dp"
            android:id="@+id/parkingdefault">
            <LinearLayout

                android:layout_width="match_parent"
                android:layout_height="match_parent"
                android:orientation="vertical"
                android:gravity="center"
                android:background="@color/colororangeplusyellow">
                <ImageView
                    android:layout_width="64dp"
                    android:layout_height="64dp"
                    android:background="@drawable/parking_default"
                    android:padding="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:textStyle="bold"
                    android:layout_marginTop="10dp"
                    android:text="Pay Park"/>
                <View
                    android:layout_width="match_parent"
                    android:layout_height="1dp"
                    android:background="@android:color/black"
                    android:layout_margin="10dp"/>
                <TextView
                    android:layout_width="wrap_content"
                    android:layout_height="wrap_content"
                    android:gravity="center"
                    android:text="Pay your Parking with us"
                    android:padding="5dp"
                    android:textColor="@android:color/black"/>

            </LinearLayout>
        </android.support.v7.widget.CardView>
    </LinearLayout>
</LinearLayout>

11.26 Android frangment_enrolfragment.xml

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.Enrolfragment"
    android:id="@+id/enrolfinal">

    <ImageView
        android:layout_width="wrap_content"
        android:layout_height="160dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_alignParentTop="true"
        android:src="@drawable/overview_header"
        android:id="@+id/imageView4" />

    <!-- TODO: Update blank fragment layout -->
    <EditText
        android:id="@+id/cardtext"
        android:layout_width="331dp"
        android:layout_height="49dp"
        android:layout_alignLeft="@+id/emailtxt"
        android:layout_alignStart="@+id/emailtxt"
        android:layout_below="@+id/emailtxt"
        android:hint="Client card Number/Username"
        android:inputType="number" />

    <EditText
        android:id="@+id/emailtxt"
        android:layout_width="332dp"
        android:layout_height="wrap_content"
        android:layout_below="@+id/imageView4"
        android:layout_centerHorizontal="true"
        android:layout_marginTop="25dp"
        android:hint="Email"
        android:inputType="textEmailAddress" />

    <EditText
        android:id="@+id/passtxt"
        android:layout_width="332dp"
        android:layout_height="wrap_content"
        android:layout_alignEnd="@+id/cardtext"
        android:layout_alignRight="@+id/cardtext"
        android:layout_below="@+id/cardtext"
        android:hint="Password"
        android:inputType="textPassword" />

    <EditText
        android:id="@+id/conpasstxt"
        android:layout_width="330dp"
        android:layout_height="wrap_content"
        android:layout_alignEnd="@+id/emailtxt"
        android:layout_alignRight="@+id/emailtxt"
        android:layout_below="@+id/passtxt"
        android:hint="Confirm Password"
        android:inputType="textPassword" />



    <TextView
        android:id="@+id/backtologintxt"
        android:layout_width="134dp"
        android:layout_height="34dp"
        android:layout_alignParentBottom="true"
        android:layout_centerHorizontal="true"
        android:layout_marginBottom="14dp"
        android:text="Back to Login"
        android:textSize="18sp"
        android:textStyle="bold"
        tools:layout_editor_absoluteX="146dp"
        tools:layout_editor_absoluteY="447dp" />

    <Button
        android:id="@+id/enrolbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/cancelbtn"
        android:layout_alignBottom="@+id/cancelbtn"
        android:layout_alignLeft="@+id/conpasstxt"
        android:layout_alignStart="@+id/conpasstxt"
        android:background="@drawable/buttonshape"
        android:text="Enrol"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/loginbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBottom="@+id/backtologintxt"
        android:layout_alignEnd="@+id/cancelbtn"
        android:layout_alignRight="@+id/cancelbtn"
        android:background="@drawable/buttonshape"
        android:text="login"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/cancelbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_above="@+id/loginbtn"
        android:layout_marginBottom="15dp"
        android:layout_toEndOf="@+id/backtologintxt"
        android:layout_toRightOf="@+id/backtologintxt"
        android:background="@drawable/buttonshape"
        android:text="@string/cancel"
        android:textColor="#FFFFFF"
        android:textSize="15sp"/>

</RelativeLayout>

11.27 Android fragment_loginfragment.xml

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.Loginfragment"
    android:id="@+id/loginfrag">


    <ImageView
        android:layout_width="match_parent"
        android:layout_height="160dp"
        android:src="@drawable/overview_header"
        android:id="@+id/imageView2" android:contentDescription="@string/todo" />

    <!-- TODO: Update blank fragment layout -->
    <EditText
        android:id="@+id/loginemail"
        android:layout_width="331dp"
        android:layout_height="49dp"
        android:layout_below="@+id/imageView2"
        android:layout_centerHorizontal="true"
        android:layout_marginTop="22dp"
        android:hint="@string/email"
        android:inputType="textEmailAddress"
        android:ems="10"/>

    <EditText
        android:id="@+id/passtxt1"
        android:layout_width="331dp"
        android:layout_height="49dp"
        android:layout_below="@+id/loginemail"
        android:layout_centerHorizontal="true"
        android:layout_marginTop="34dp"
        android:hint="@string/password"
        android:inputType="textPassword"
        android:ems="10"/>


    <Switch
        android:id="@+id/savepass"
        android:layout_width="wrap_content"
        android:layout_height="wrap_content"
        android:layout_alignEnd="@+id/passtxt1"
        android:layout_alignRight="@+id/passtxt1"
        android:layout_below="@+id/passtxt1"
        android:text="@string/remember_me" />


    <TextView
        android:id="@+id/notclienttxt"
        android:layout_width="wrap_content"
        android:layout_height="wrap_content"
        android:layout_alignBottom="@+id/enrolbtn1"
        android:layout_alignEnd="@+id/loginbtn1"
        android:layout_alignRight="@+id/loginbtn1"
        android:text="@string/not_a_client" />


    <Button
        android:id="@+id/loginbtn1"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignLeft="@+id/passtxt1"
        android:layout_alignStart="@+id/passtxt1"
        android:layout_below="@+id/savepass"
        android:background="@drawable/buttonshape"
        android:layout_marginTop="48dp"
        android:text="@string/login"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/fingerprintbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/loginbtn1"
        android:layout_alignBottom="@+id/loginbtn1"
        android:layout_alignEnd="@+id/savepass"
        android:layout_alignRight="@+id/savepass"
        android:background="@drawable/buttonshape"
        android:text="@string/fplogin"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/enrolbtn1"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_below="@+id/loginbtn1"
        android:layout_centerHorizontal="true"
        android:background="@drawable/buttonshape"
        android:text="@string/enrol"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />
</RelativeLayout>

11.29 Android fragment_overview.xml

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:tools="http://schemas.android.com/tools"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.Overview"
    android:id="@+id/overviewlayout">

    <!-- TODO: Update blank fragment layout -->
    <ImageView
        android:id="@+id/imageView"
        android:layout_width="match_parent"
        android:layout_height="160dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_alignParentTop="true"
        android:src="@drawable/overview_header" />

    <TextView
        android:id="@+id/cardnumber"
        android:layout_width="match_parent"
        android:layout_height="40dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_below="@+id/imageView"
        android:layout_marginTop="19dp"
        android:text="Account Number"
        android:textColor="@android:color/black"
        android:textSize="28sp"
        android:textAlignment="center"
        android:textAllCaps="true"
        android:background="@color/colorlightcyan"
        android:textStyle="bold"/>

    <TextView
        android:id="@+id/displaynumber"
        android:layout_width="match_parent"
        android:layout_height="40dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_below="@+id/cardnumber"
        android:layout_marginTop="17dp"
        android:textColor="@android:color/black"
        android:textSize="28sp"
        android:hint="account number "
        android:textAlignment="center"
        android:textStyle="bold"/>

    <TextView
        android:id="@+id/overtxt"
        android:layout_width="match_parent"
        android:layout_height="40dp"
        android:layout_alignParentEnd="true"
        android:layout_alignParentLeft="true"
        android:layout_alignParentRight="true"
        android:layout_alignParentStart="true"
        android:layout_below="@+id/displaynumber"
        android:layout_marginTop="20dp"
        android:ems="10"
        android:text="show Balance"
        android:textColor="@android:color/black"
        android:textSize="28sp"
        android:textAlignment="center"
        android:textAllCaps="true"
        android:background="@color/colorlightcyan"
        android:textStyle="bold"/>

    <TextView
        android:id="@+id/displaybal"
        android:layout_width="match_parent"
        android:layout_height="40dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_below="@+id/overtxt"
        android:layout_marginTop="15dp"
        android:hint="balance display"
        android:textColor="@android:color/black"
        android:textSize="28sp"
        android:textAlignment="center"
        android:textStyle="bold"/>

    <Button
        android:id="@+id/logoutbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_below="@+id/displaybal"
        android:layout_marginLeft="29dp"
        android:layout_marginStart="29dp"
        android:layout_marginTop="21dp"
        android:background="@drawable/buttonshape"
        android:text="Logout"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/cancelbtnover"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/logoutbtn"
        android:layout_alignBottom="@+id/logoutbtn"
        android:layout_alignParentEnd="true"
        android:layout_alignParentRight="true"
        android:layout_marginEnd="31dp"
        android:layout_marginRight="31dp"
        android:background="@drawable/buttonshape"
        android:text="Back"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />
</RelativeLayout>

11.30 Android fragment_pay.xml

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    xmlns:tools="http://schemas.android.com/tools"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.transfer">

    <com.aldoapps.autoformatedittext.AutoFormatEditText
        android:id="@+id/paytxt"
        android:layout_width="300dp"
        android:layout_height="wrap_content"
        android:layout_alignParentTop="true"
        android:layout_centerHorizontal="true"
        android:layout_marginTop="40dp"
        android:hint="Enter the Amount here:"
        android:maxLength="8"
        app:isDecimal="true" />

    <View
        android:layout_width="match_parent"
        android:layout_height="3dp"
        android:layout_below="@+id/paytxt"
        android:id="@+id/viewid"/>

    <LinearLayout
        android:layout_width="match_parent"
        android:layout_height="400dp"
        android:layout_below="@+id/viewid"
        android:id="@+id/linearlayout"
        android:orientation="horizontal"
        android:layout_centerInParent="true">


    </LinearLayout>

    <Button
        android:id="@+id/paybtn"
        android:layout_width="118dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/btn12"
        android:layout_alignBottom="@+id/btn12"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_marginLeft="37dp"
        android:layout_marginStart="37dp"
        android:background="@drawable/buttonshape"
        android:text="Pay"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />


    <Button
        android:id="@+id/btn12"
        android:layout_width="118dp"
        android:layout_height="45dp"
        android:layout_alignParentEnd="true"
        android:layout_alignParentRight="true"
        android:layout_below="@+id/linearlayout"
        android:layout_marginEnd="63dp"
        android:layout_marginRight="63dp"
        android:layout_marginTop="55dp"
        android:background="@drawable/buttonshape"
        android:text="Back"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <ImageView
        android:id="@+id/img"
        android:layout_width="match_parent"
        android:layout_height="400dp"
        android:layout_alignLeft="@+id/linearlayout"
        android:layout_alignStart="@+id/linearlayout"
        android:layout_below="@+id/viewid"
        android:background="@color/lightgray" />
</RelativeLayout>

11.31 Android fragment_transfer.xml

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:tools="http://schemas.android.com/tools"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    android:layout_width="match_parent"
    android:layout_height="match_parent"
    tools:context="com.example.susha.qrmoney.transfer">

    <!-- TODO: Update blank fragment layout -->

    <EditText
        android:id="@+id/totxt"
        android:layout_marginTop="40dp"
        android:layout_width="331dp"
        android:layout_height="wrap_content"
        android:layout_alignLeft="@+id/mini"
        android:layout_alignParentTop="true"
        android:layout_alignStart="@+id/mini"
        android:hint="To:"
        android:inputType="textEmailAddress" />



    <com.aldoapps.autoformatedittext.AutoFormatEditText
        android:id="@+id/mini"
        android:layout_width="331dp"
        android:layout_marginTop="20dp"
        android:layout_height="wrap_content"
        android:layout_below="@+id/totxt"
        android:layout_centerHorizontal="true"
        android:hint="Enter the Amount here:"
        android:maxLength="8"
        app:isDecimal="true" />

    <View
        android:layout_width="match_parent"
        android:layout_height="3dp"
        android:layout_below="@+id/mini"
        android:id="@+id/view"/>
    <LinearLayout
        android:layout_width="match_parent"
        android:layout_height="300dp"
        android:id="@+id/linearlayout1"
        android:layout_below="@+id/view"
        android:orientation="horizontal"
        android:layout_centerInParent="true"
        android:background="@color/lightgray">

        <ImageView
            android:id="@+id/transferimage"
            android:layout_width="match_parent"
            android:layout_height="300dp" />

    </LinearLayout>


    <Button
        android:id="@+id/gnbtn"
        android:layout_width="100sp"
        android:layout_height="45dp"
        android:layout_alignParentBottom="true"
        android:layout_alignParentLeft="true"
        android:layout_alignParentStart="true"
        android:layout_marginBottom="48dp"
        android:layout_marginLeft="33dp"
        android:layout_marginStart="33dp"
        android:background="@drawable/buttonshape"
        android:text="Generate"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/btn13"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/transferbtn"
        android:layout_alignBottom="@+id/transferbtn"
        android:layout_alignParentEnd="true"
        android:layout_alignParentRight="true"
        android:layout_marginEnd="30dp"
        android:layout_marginRight="30dp"
        android:background="@drawable/buttonshape"
        android:text="Back"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />

    <Button
        android:id="@+id/transferbtn"
        android:layout_width="100dp"
        android:layout_height="45dp"
        android:layout_alignBaseline="@+id/gnbtn"
        android:layout_alignBottom="@+id/gnbtn"
        android:layout_centerHorizontal="true"
        android:background="@drawable/buttonshape"
        android:text="Transfer"
        android:textColor="#FFFFFF"
        android:textSize="15sp" />
</RelativeLayout>

11.32 Android nav_header_main.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:app="http://schemas.android.com/apk/res-auto"
    android:layout_width="match_parent"
    android:layout_height="@dimen/nav_header_height"
    android:background="@drawable/header"
    android:gravity="bottom"
    android:orientation="vertical"
    android:paddingBottom="@dimen/activity_vertical_margin"
    android:paddingLeft="@dimen/activity_horizontal_margin"
    android:paddingRight="@dimen/activity_horizontal_margin"
    android:paddingTop="@dimen/activity_vertical_margin">

    <ImageView
        android:id="@+id/logoimage"
        android:layout_width="120dp"
        android:layout_height="120dp"
        android:paddingTop="@dimen/nav_header_vertical_spacing"
        app:srcCompat="@drawable/emoney" />



    <TextView
        android:id="@+id/logotext"
        android:layout_width="wrap_content"
        android:layout_height="25dp"
        android:text="QR Banking"
        android:textColor="@color/colorBlue"/>
</LinearLayout>

11.28 Android slide.xml

<?xml version="1.0" encoding="utf-8"?>
<LinearLayout
    xmlns:android="http://schemas.android.com/apk/res/android" android:layout_width="match_parent"
    android:layout_height="match_parent"
    android:orientation="vertical"
    android:gravity="center"
    android:id="@+id/slidelinearlayout">


    <ImageView
        android:layout_width="200dp"
        android:layout_height="200dp"
        android:id="@+id/slideimg"/>

    <TextView
        android:layout_width="wrap_content"
        android:layout_height="wrap_content"
        android:text="Title Here"
        android:textColor="#fff"
        android:textSize="30dp"
        android:textStyle="bold"
        android:layout_marginTop="40dp"
        android:id="@+id/txttitle"
        />

    <TextView
        android:layout_width="wrap_content"
        android:layout_height="wrap_content"
        android:textColor="#fff"
        android:textSize="17dp"
        android:layout_marginTop="40dp"
        android:textAlignment="center"
        android:id="@+id/txtdescription"
        android:text="Description Here"
        />

<Button
    android:layout_width="wrap_content"
    android:layout_height="wrap_content"
    android:text="Start"
    android:id="@+id/btnstart"
    android:textSize="15dp"
    android:layout_marginTop="40dp"
    android:background="@drawable/buttonshape"
    android:textColor="#fff"/>
</LinearLayout>

11.28 Android activity_main_drawer.xml

<?xml version="1.0" encoding="utf-8"?>
<menu xmlns:android="http://schemas.android.com/apk/res/android"
    xmlns:tools="http://schemas.android.com/tools"
    tools:showIn="navigation_view">

    <group android:checkableBehavior="single">
        <item
            android:id="@+id/dashboard"
            android:icon="@drawable/overview"
            android:title="DashBoard" />
        <item
            android:id="@+id/pay"
            android:icon="@drawable/pay"
            android:title="Pay" />
        <item
            android:id="@+id/transfer"
            android:icon="@drawable/transfer"
            android:title="Transfer" />
        <item
            android:id="@+id/wallet"
            android:icon="@drawable/wallet"
            android:title="Wallet" />
        <item
            android:id="@+id/logoutbtn"
            android:icon="@drawable/wallet"
            android:title="Logout" />
    </group>

</menu>